## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Theory of Structures-II(FE702)/(4023) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | State and explain Mohr’s theorems for slope and deflection of beam with an example. |  |
| b) | What do you mean by a fixed beam? How the Bending Moment diagram of such a beam can be drawn? |  |
| c) | Define stiffness and relative stiffness of a member in Moment Distribution Method. Give examples. |  |
| d) | What are short columns and long columns? Explain buckling of long columns and define crippling load. |  |
| e) | In three hinged arch of span ‘l’ and rise ‘h’ carrying a uniformly distributed load of ‘W’ N/m over the whole span, show that the horizontal thrust is wl28h |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A cantilever AB, fixed at A and free at B of span ‘l’ carries a uniformly distributed load W per unit run. Find the slope at B and deflection at B. |  |
| b) | A simply supported beam of span ‘l’ carries a central point load W. Find the slope at the support and the deflection at mid span. |  |
| c) | A propped cantilever of length ‘l’ of 6m carries a uniformly distributed load of 10 KN/m over the entire span. If the prop at free end is at the same level as fixed end, find the reaction of the prop. Draw the BMD. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A fixed beam of 6m span is carrying a point load of 10KN at a distance of 2m from the left hand support. Draw the SFD and BMD of the beam. |  |
| b) | A fixed beam of 4m span is carrying a u.d.l. of 10KN/m throughout the span. Draw the SFD and BMD. |  |
| c) | A fixed beam of 3m span carries two point loads of 10KN each at a distance of 1m and 2m from the left support. Draw the SFD and BMD of the beams. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A continuous beam ABC consists of two spans AB=4m and BC=3m, the end A being fixed. The span AB carries a point load of 8KN at 1m from A while the span BC carries a point load of 6KN at 1m from C. Find the support moments and draw BM diagram. Take EI to be same for both spans. |  |
| b) | A continuous beam ABC consists of two span AB=BC=6m, the ends A & C are fixed. Span AB carries a point load of 8KN at mid span and span BC carries uniformly distributed load of 5KN/m over entire span. Draw BM diagram. EI is constant. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | A portal frame ABCD is fixed at A & D, AB=CD=4m and horizontal BC=2m. Span BC carries uniformly distributed load of 20KN/m. EI is uniform for entire frame. Analyze the frame using moment distribution method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel column is of length 8m and diameter 600mm with both ends hinged. Determine the crippling load by Euler’s formula. Take E=2.1x105N/mm2. |  |
| b) | A steel tube 4m long, 3m internal diameter and 4mm thick is used as compression member with both ends built in. Find the crippling load. E = 2x105N/mm2 |  |
| c) | A rolled steel joist ISMB 300 is to be used as a column of 3m length with both ends fixed. Find the safe axial load on the column. Take factor of safety 3, fc = 320N/mm2,  = 1/7500. Properties of column section: Area=5626mm2 , Ixx = 8.603x107 mm4 and Iyy = 4.539x107 mm4. |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Three hinged arch of span 40m and rise 10m carries concentrated loads 200KN and 150KN at distances 8m and 16m from the left end and a uniformly distributed load of 50KN/m on the right half of the span. Find the horizontal thrust. |  |
| b) | Three hinged parabolic arch ACB has a span of 50m. Its abutments A and B are at depths 16m and 25m below the crown C. The arch carries a concentrated load of 100KN on AC at distance of 10m from C and a concentrated load of 200KN on BC at a distance of 20m from C. Calculate the vertical and horizontal reactions at A and B. |  |

***********