## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/EI

Subject: Analog Communication(EN402) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the block diagram of a typical communication system. |  |
| b) | Define frequency modulation. Draw the FM waveforms. |  |
| c) | Draw the frequency spectrum of AM and explain. |  |
| d) | State the main features of vestigial sideband system. |  |
| e) | Draw and describe the significance of noise triangle in FM. |  |
| f) | Define modulation index in FM. |  |
| g) | Draw the circuit of a balanced slope detector. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the reasons why modulation is needed. |  |
| b) | Derive the equation for an AM wave. |  |
| c) | Draw the block diagrams of low-level AM transmitter & high- level AM transmitter. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram, explain working of a balanced modulator circuit using FETS. |  |
| b) | With a neat circuit diagram, describe how AM demodulation is performed. |  |
| c) | A 600W carrier is modulated to a depth of 75%. Calculate the total power in this amplitude–modulated wave. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between narrowband and wideband FM. |  |
| b) | State the advantages of FM over AM. |  |
| c) | With a neat circuit diagram, explain working of a varactor diode modulator circuit. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Draw the block diagram of a stereophonic FM transmitter. ii) Draw the block diagram of FM receiver. Explain functions of any two blocks not present in an AM Receiver. |  |
| b) | Define the following with respect to antennas: - Antenna gain ii) beamwidth |  |
| c) | Why is AGC needed? Describe working of a delayed AGC circuit. |  |
| Q.No.6 | Write short notes on any three: | 3 x 4 = 12 |
| a) | Internal noises |  |
| b) | Coaxial cables |  |
| c) | Parabolic reflector antenna with horn feed |  |
| d) | Signal to Noise ratio |  |

***********