## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Manufacturing Process-I(4034) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the different materials used for preparing pattern and also mention the factors, which govern the selection of a proper material for pattern making. |  |
| b) | List any six types of moulding tools and explain any one. |  |
| c) | What is Honey-combing in castings? |  |
| d) | What is welding? Give difference between Arc welding and Gas welding. |  |
| e) | What is cold drawing? |  |
| f) | What is sintering in Powder metallurgy? |  |
| g) | What are core prints? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Why patterns are provided with allowances? List the various allowances and explain any one. |  |
| b) | With the help of sketch explain the construction of any two types of patterns. |  |
| c) | Explain the various properties of moulding sand. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain core making. |  |
| b) | With a neat sketch, explain the working of a cupola furnace. |  |
| c) | Explain permanent mould casting. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List and sketch the various positions in welding of plates and pipes. |  |
| b) | Explain TIG welding with a neat sketch. |  |
| c) | With the help of sketch explain Oxy-Acetylene gas welding. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give at least six differences between Hot working and Cold working. |  |
| b) | Write short note on: i) Cold bending ii) Hot spinning. |  |
| c) | Giver advantages and limitations of Powder metallurgy. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Blending of powders |  |
| b) | Special moulding processes |  |
| c) | Spot welding |  |
| d) | Molten metal handling equipments |  |
| e) | “Piercing” in metal forming process |  |

***********