## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Shipbuilding Engg.

Subject: Basic Ship Theory-II(SH401)/(4335) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Explain any five of the following: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Dynamic Stability |  |
| b) | Permeability |  |
| c) | Stowage factor |  |
| d) | Use of Bonjean curves in launching |  |
| e) | Load and Shear force curve |  |
| f) | Compressive and Tensile stresses |  |
| g) | GRT and NRT (only for students of old scheme) |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | How is weight curve arrived at in ship strength calculations? | (6) |
| b) | Explain how the section modulus of midship section can be improved when needed. | (6) |
| Q.No.3. | Using the table of KN ordinates below calculate the righting levers for a ship when her displacement is 40,000 tonnes and her actual KG is 10m. Draw the resulting statical stability curve and from it, determine: i) Maximum GZ Value ii) Approximate GM iii) Righting moment at the angle of heel of 25 iv) Range of stability. | (12) |
| Angle of heel; 0; 5; 10; 15; 20; 30; 45; 60; 75; 90. KN ordinates; 0; 0.90; 1.92; 3.11; 4.25; 6.30; 8.44; 9.39; 9.29; 8.50. |
| Q.No.4. | A box shaped vessel 75m long x 10m wide x 6m deep is floating in salt water on an even keel at a draft of 4.5m. Find the new drafts if a forward compartment 5m long is bilged. | (12) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Describe in short the procedure of end launching of a ship with neat sketch. | (6) |
| b) | Draw a set of typical end launching curves. | (6) |
| Q.No.6. | A uniform box shaped barge 40m x 12m beam is divided into four cargo holds each 10m long. The barge is loaded with 600 t of iron ore and level stored as follows: No.01  135t, No.02  165t, No.3  165t, No.04  135t. The loaded barge floats on an even keel draught of 1.75m. Construct a curve of shear force and bending moment for this loading condition. | (12) |
| Q.No.7. | Answer the following Questions: |  |
| a) | Determine the dynamic stability of a ship of displacement 4000 tonnes and having the following GZ values till 40 heel. Angle of heel (); 0; 10; 20; 30; 40. GZ (metres); 0; 0.09; 0.2; 0.45; 0.4. | (9) |
| b) | Explain bale capacity of a ship. | (3) |
|  |  | …..2/- |
|  | -2- |  |
|  | Only for students of Old scheme: |  |
| Q.No.8. | Answer the following Questions: |  |
| a) | What is the purpose of tonnage certificate in ship’s day to day operations? | (6) |
| b) | Determine the gross tonnage and net tonnage of a vessel having total enclosed volume 4500m3 of which 3600m3 is cargo volume. Total crew on board is 9. The depth of vessel is 4.5m and draft is 3m. | (6) |

***********