## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/EI

Subject: 8051 Microcontroller(EN501) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare µc 8031, 8051, 8751, 8748 with respect to RAM. ROM & I/O lines. |  |
| b) | What is the basic difference between Microprocessor & Microcontroller? |  |
| c) | What are the different flags in PSW register? |  |
| d) | How many interrupt signals are available for 8051? What are those? |  |
| e) | Give example of bit level & byte level & rotate instruction. |  |
| f) | Explain RETI instruction with example. |  |
| g) | What are the different modes of 8051 timer? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain oscillator circuit of 8051. What is the standard frequency used for µc 8051? ii) Explain PC & DPTR register of 8051. |  |
| b) | Explain port1 circuits of 8051 in detail. |  |
| c) | Explain stack & its related instructions with example. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain serial data transmission modes in detail. |  |
| b) | Explain importance of RI & TI flags in serial communication using 8051 µc. |  |
| c) | Explain special function registers IE & IP. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain following instructions: - MOVXA, @ RP - MOVX @DPTR, A - MOVC A, @ A+DPTR - MOVC A, @ A+PC |  |
| b) | Explain all rotate instructions. |  |
| c) | Explain CALL & RET instruction with suitable sample program. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program to multiply data in RAM (22)H by data in External RAM location (0015)Η. Put the LSB result to (0019)H & MSB result to (001A)H. |  |
| b) | Write a program to add numbers in internal RAM 25H, 26H & 27H. Put the result to 31H(MSB) & 30H (LSB) |  |
| c) | Write a program to split the 8 bit data which is at internal RAM (17)H into two nibbles. Store lower nibble to (18)H & higher nibble in (19)H in internal RAM. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain register addressing mode & indirect addressing mode with example. |  |
| b) | Write software time delay program for 1 msec. Assume crystal frequency = 11.0592 MHz. |  |
| c) | Explain Exchange instructions with examples. |  |

***********