## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Shipbuilding Engg.

Subject: Introduction To Thermodynamics(SH404)/(4334) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is meant by thermodynamic medium, system and surrounding? |  |
| b) | Draw the diesel cycle on the PV diagram and name the processes. |  |
| c) | Explain the terms as referred to steam: i) Enthalpy of water ii) Enthalpy of evaporation iii) Superheat. |  |
| d) | Indicate the importance of heat transfer and different modes of heat transfer. |  |
| e) | State the first law of thermodynamics and state its importance. |  |
| f) | What is subcooling and what is its advantage? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and explain the Zeroth’s law of thermodynamics. |  |
| b) | Discuss the characteristics Gas Equation. The pressure of a gas supplied to an engine is measured as 101.756 KPa. Determine the volume of 1.8Kg of this gas if its temperature is 85C. The gas constant of the gas is 0.686 KJ/KgK. |  |
| c) | What are the basic gas processes? 0.3m3 of a gas filled in a closed tank at an initial condition of 2.75 bar and 40C. The gas is heated until the pressure in the tank becomes 4 bar. Find the change in internal energy of the gas. Assume R = 0.287 KJ/KgK, Kv = 0.711 KJ/KgK for gas. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Two Kg of air at 150KPa and 30C undergo a constant pressure process until the volume is trippled. Determine i) Change in internal energy ii) Change in enthalpy. Take Kp=1.0035 KJ/KgK and Kv=0.7165 KJ/KgK |  |
| b) | What is an adiabatic process? A perfect gas is compressed adiabatically from a state of 1.93 MPa, volume 0.1m3 and temperature -4C, to a pressure of 5.84 MPa. Find i) Temperature and volume at the end of compression ii) The change in enthalpy. Take Kp=1.0005 KJ/KgK and Kv =0.718 KJ/KgK. |  |
| c) | With a simple sketch explain the working principle of a four stroke cycle diesel engine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is volumetric efficiency? What is the benefit of high volumetric efficiency? Give the difference between two stroke and four stroke cycle engines. |  |
|  |  | ……..2/- |
|  | -2- |  |
| b) | The following data and test results refer to a four-stroke cycle oil engine: Number of cylinders = 4;bore = 10cm; Stroke = 15cm; Indicated mean effective pressure = 670KPa; Speed = 34rps; brake torque = 185Nm; oil consumption 12.89 Kg/hr; caloric value of fuel oil =42,000 KJ/Kg. Determine i) Indicated power ii) Brake power iii) Brake thermal efficiency. |  |
| c) | What is the need of lubricating system in the IC engine? An engine uses 6.5Kg of oil per hour of calorific value 30,000 KJ/Kg. If the Brake power of the engine is 22KW and mechanical efficiency 85%, calculate the Indicated thermal efficiency and Brake thermal efficiency |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of simple sketches, explain the formation of steam at constant pressure. |  |
| b) | Determine the condition of steam in each of the following cases: - At a pressure of 10 bar and temperature 200C - At a pressure of 8 bar and volume 0.22 m3/kg - At a pressure of 12 bar, if 2,688 KJ/Kg are required to produce it from water at 0C. |  |
| c) | Explain the vapour compression refrigeration cycle with the help of P-H chart. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List a few important factors, which govern the transfer of heat by conduction. A furnace wall is made of refractory bricks of 350mm thick. The inner and outer surfaces of the wall have temperature of 900C and 100C. Find the heat loss per square metre per hour. Assume K = 4.5W/mK for refractory bricks. |  |
| b) | State a few applications of refrigeration on ships. List the desirable characteristics of a good refrigerant. |  |
| c) | A refrigeration machine using R12 as refrigerant operates between -7C and 36C, the vapour is dry at the entry of compressor and there is no undercooling in the condensor. The vapour leaves as refrigerant gas at 46C from compressor. |  |
| Sat. Temp C; Pressure (bar); Enthalpy liquid (KJ/Kg); Enthalpy vapour (KJ/Kg); Entropy of Sat. Vapour (KJ/KgK). 36C; 9 bar; 456.4; 585.3; 4.74. -7C; 2.5 bar; 412.4; 570.3; 4.76. |
|  | Take Cp of vapour at 9 bar as 0.67 KJ/KgK. Determine the COP of the cycle. |  |

***********