## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg.

Subject: Design of Structures-II(Steel) [CE602] [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS 800-2007 and Steel tables is permissible.

5) Take Fe410 Steel with fy =250 MPa

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | State the advantages and disadvantages of construction in structural steel. |  |
| b) | Draw neat sketches of the different types of welds and their welding symbols. |  |
| c) | List the type of cross section that can be used as tension members and their use in typical structures. |  |
| d) | State the different steps to be followed while designing a slab base. |  |
| e) | What is meant by effective length of a beam member? |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Select a suitable angle section to carry a factored tensile force of 150KN. Also design suitable welded end connection. Assume design strength as fy=250N/mm2 | (9) |
| b) | Design a connection to join two plates of size 200 x 10mm of grade Fe410 to mobilize full plate tensile strength using shop fillet welds. Use lap joint. | (6) |
|  | ### OR |  |
| b) | A tie member of truss consisting of an angle section ISA 65 x 65 x 6 of Fe410 is welded to an 8mm gusset plate. Design a weld to transmit a load equal to the full strength of the member. Assume shop weld. | (6) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Design a single unequal angle strut 3.50m long to carry a factored load of 120KN. It is connected by its longer leg to a 8mm thick gusset plate. Design the end connection using fillet weld. Take fy = 250MPa. | (9) |
| b) | Design a double angle discontinuous strut to carry factored axial load of 170KN. The length of the strut between center to center of intersection is 4.0m. Take fy = 250MPa. | (6) |
|  | ### OR |  |
| b) | Design a slab base for a built up column consisting of two channels (ISLC 350) back to back separated by a distance of 220mm and carrying a factored load of 1800KN. Concrete grade M10, steel Fe410, Bearing capacity of soil 350KN/m2. | (6) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Design a simply supported beam of span 4m and loaded by a udl of 40KN/m over the whole span. Assume that the compression flange is fully restrained laterally. Take fy=250MPa. | (9) |
| b) | A cantilever beam of span 5m is subjected to a service live load of 22.5KN at its free end. Use steel with fy=250MPa and select a rolled section. There is no limit on deflection. | (6) |
|  | ### OR |  |
| b) | Compute the moment carrying capacity and shear strength of a laterally supported ISMB 300 member of length 3m. Take fy = 250MPa. | (6) …….2/- |
|  | -2- |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | Calculate the uniformly distributed live load for a roof with slope of 18º. | (3) |
| b) | How is the spacing of purlins fixed? | (3) |
| c) | Design an I section purlin for an industrial building to support a 18 guage galvanized corrugated iron sheet roof (with a dead weight of 131 N/m2) with the following data. Spacing of truss c/c =5.0m Span of truss =15m Spacing of purlin c/c =1.5m Intensity of wind pressure = 2.3KN/m2 Yield stress of steel = 250Mpa | (9) |

***********