## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Computer Engg./E&EE

Subject: Introduction To Microcontrollers(CO512) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a schematic showing microcontroller 8051 connected to single LED at any port. |  |
| b) | List some features of PIC microcontrollers. |  |
| c) | Draw block diagram of Harvard architecture. Give some of its advantages. |  |
| d) | Explain increment instruction with examples. |  |
| e) | Write instructions to set bits 1, 3, 5 of port 0. |  |
| f) | What will be content of A, B, OV flag at the end of execution of following instructions: MOV A, # 7Bh MOV 0F0h, # 02h MUL AB |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are interrupts? Explain steps taken by µc 8051 in executing an interrupt. |  |
| b) | What are addressing modes? Explain 3 addressing modes with examples. |  |
| c) | Explain addition and subtraction instructions with reference to µc 8051. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain features of 4 bit and 8 bit microcontrollers. |  |
| b) | Give pin description of 8051 with help of neat diagram. |  |
| c) | What are subroutines? Explain steps taken by 8051 while executing CALL instruction. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program to perform the following mathematic operation (2x2 + 7). Consider ‘x’ to be any number of your choice. |  |
| b) | Write a note on unconditional jump in 8051. |  |
| c) | Draw a schematic showing connection of external memory ROM and RAM to µc 8051. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between microcontrollers and microprocessors. |  |
| b) | Draw and explain schematic to interface 8 bit ADC to µc 8051. |  |
| c) | With the help of schematic describe interfacing of 2x2 matrix keyboard to 8051. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain timer mode 0 and 1 of µc 8051. |  |
| b) | Draw and explain schematic to interface 7-segment LED display in common cathode at port 0 of µc 8051. |  |
| c) | Write a note on ATMEGA (AVR) microcontrollers. |  |

***********