## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/EI/E/E&EE

Subject: Electronic Instrumentation(EN404) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the block diagram of general instrumentation system and explain in brief. |  |
| b) | State one example of i) Resistive transducer ii) Temperature measuring transducer iii) Optical transducer. |  |
| c) | List three advantages of magnetic flowmeter. |  |
| d) | State three characteristics of Instrumentation amplifier. |  |
| e) | Explain pressure measurement using strain guage. |  |
| f) | List three application areas of SCADA. |  |
| g) | Draw the block diagram of Data Acquisition System. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain classification of transducers giving an example of each. |  |
| b) | Explain four characteristics of ideal transducer. |  |
| c) | Explain speed measurement using photoelectric pick up. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how capacitive transducer attains variation in capacitance by i) Varying of distance ii) Varying of area. |  |
| b) | Briefly explain operation of i) LDR ii) Photodiode. |  |
| c) | Explain the operation of LVDT with the help of neat diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain float operated resistive method for level measurement. |  |
| b) | Explain flow measurement using turbine flowmeter. |  |
| c) | Explain vibration measurement using piezoelectric accelerometer. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain block diagram of strip chart recorder |  |
| b) | Explain bridge circuit for RTD. |  |
| c) | List three advantages of LED and three advantages of LCD. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Application areas of Data Acquisition System |  |
| b) | SCADA system |  |
| c) | Process Control System |  |
| d) | Thermistor |  |

***********