## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Automobile Engg.

Subject: Auto Engines-I(AT402) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the purpose of vibration dampers. |  |
| b) | What is the purpose of cooling system in IC engine? |  |
| c) | Explain the purpose of oil level indicator in a lubrication system. |  |
| d) | What are cylinder liners? Give its functions with sketch. |  |
| e) | Explain the causes for an overheating engine. |  |
| f) | List out the possible causes of low Lubricating oil pressure. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain working of timing pulley and list out advantages over timing gears. |  |
| b) | With a neat sketch explain working of exhaust manifold and its design consideration. |  |
| c) | Explain with a neat sketch the construction of piston and advantages of Aluminium alloy pistons. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the purpose of i) Water pump ii) Thermostat in cooling system. |  |
| b) | Explain the working of radiator cap with neat sketch. |  |
| c) | Troubleshoot the causes in cooling system. Provide suitable remedies for the same. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List out 6 lubricating properties and explain any two in detail. |  |
| b) | With a neat sketch explain the construction & operation of wet sump lubrication system. |  |
| c) | Explain the construction & working of a lubrication oil gauge. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Name and explain the characteristics of good gasoline fuel. |  |
| b) | List the characteristics of good diesel fuel & explain the term Cetane rating. |  |
| c) | Explain with sketch the working of simple carburetor. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | List out four points of difference between 2 stroke & 4 stroke engine. |  |
| b) | Explain the different types of combustion chambers in SI engine with sketch. |  |
| c) | Write a note on Pre-ignition. |  |
| d) | Describe with sketch pump cooling system. |  |
| e) | Write a note on lubrication system contamination. |  |

***********