## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg./CT

Subject: Quantity Surveying & Costing(4014) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the data required to prepare an estimate. |  |
| b) | State the units of measurement for the following items of work: i) Varnishing ii) Fixing wash basin iii) Steel Reinforcement |  |
| c) | What is ‘Schedule on Rates’? |  |
| d) | Define taskwork. |  |
| e) | What are credentials of a valuer? |  |
| f) | What is purpose of inviting tenders? |  |
| g) | State the purpose of writing specifications. |  |
| Q.No.2. | Answer any one of the following Questions: | (15) |
| a) | A plan and wall section of a single storeyed structure is shown in Fig.1. Work out the quantities of any five of the following items: - Earthwork in excavations in foundation trenches - Laterite stone masonry in foundation and plinth in cm: 1:6 - Brick masonry in superstructure 30cm thick in cm 1:6 - R.C.C slab M20 12 cm thick - External plaster in double coat cm:1:4, 18mm thick - Internal painting in O.B.D - Ceramic tile flooring |  |
| b) | Prepare an estimate for the portion of a road between chainage 14 to 22 from the data given below. Draw also the longitudinal and cross-sectional profile. Turfing with grass sols shall be provided for sides of embankments @Rs.30.00 per sq.m. Apply market rates of earthwork in cutting and embankment. The formation width of proposed road is 12m, side slopes 1.5:1 in cutting and 2:1 in banking. |  |
| Chainage 30m; 14; 15; 16; 17; 18; 19; 20; 21; 22. RL of Ground; 108.60; 109.25; 109.40; 108.85; 108.50; 107.25; 106.8; 107.19; 107.20. |
|  | The road formation is proposed at uniform falling gradient 1 in 200 passing through CL at chainage 14. Length of one chain=30m. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Carry out rate analysis for R.C.C 1:1.5:3 including centering and shuttering but excluding steelwork for 10m3. |  |
| b) | Carry out rate analysis for 23cm laterite stone masonry in superstructure in cm:1:4 for 10m3 |  |
|  |  | …..2/- |
|  | -2- |  |
| c) | Explain the method of preparing an approximate estimate in respect of following: - For a Highway ii) Water supply project |  |
| d) | What is a contract document? What are its important components? |  |
| Q.No.4. | Answer any three of the following Questions: | 3 x 5 = 15 |
| a) | Draft a tender notice for inviting tenders for construction of a school building at an estimated cost of Rs.65 lakhs to be completed in 18months. |  |
| b) | Explain the terms i) Cash imprest ii) temporary advance. |  |
| c) | i) What is meant by valuation? State the purpose of elevation. ii) State the factors affecting the value of an asset. |  |
| d) | Differentiate between the following: - Material contract V/s Labour contract - Earnest money Deposit V/s Security Deposits |  |
| Q.No.5. | Write short notes on any five: | 5 x 3 = 15 |
| a) | Types of specifications |  |
| b) | Daily labour v/s Piece work |  |
| c) | Arbitration |  |
| d) | Termination of contracts |  |
| e) | Essentials of a valid contract |  |
| f) | Supplementary v/s Revised estimate |  |
| g) | Classification of value |  |