## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics & Communication Engg.

Subject: Satellite & Optical Fiber Communication(EN605) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe three advantages of optional fibers over copper cables. |  |
| b) | Define the following: i) Acceptance angle ii) Numerical aperture. |  |
| c) | Describe any two types of connectors used with optional fibers. |  |
| d) | Distinguish between LEO, MEO and GEO satellite orbits. |  |
| e) | Explain the need for a power subsystem in satellite systems. |  |
| f) | Describe three advantages of CDMA over TDMA and FDMA techniques. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe and contrast the following optical fibers: i) Multimode step-index fiber ii) Graded-index fiber. |  |
| b) | Describe the operation of optical fiber drawing apparatus with the help of a neat diagram. |  |
| c) | Describe any three losses that occur in optical fibers and means by which these losses can be minimized or overcome. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat diagram and explain the principle of operation of an injection laser diode. |  |
| b) | With the help of a neat diagram, explain the principle of operation of a PIN diode. Compare and contract it with an avalanche photodiode. |  |
| c) | Describe any two types of splices. Explain losses due to splicing and methods used to minimize these losses. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram and describe the operation of the altitude and orbit control subsystem in a satellite system. |  |
| b) | With the help of a block diagram, describe a single-channel satellite transponder. |  |
| c) | Describe the functions of the telemetry, tracking and command subsystem in a satellite system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in detail, the CDMA technique used in satellite communication. |  |
| b) | Why are multiple access techniques used in satellite communication? Distinguish between all such techniques. |  |
| c) | With the help of a diagram describe the construction of an optical fiber and explain how low light travels through an optical fiber. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer the following Questions: | 3 x 4 = 12 |
| a) | Describe four characteristics to be possessed by photodetectors used in optical fiber communication. |  |
| b) | Describe the geosynchronous satellite orbit. |  |
| c) | Write a short note on the communication subsystem of a satellite. |  |

***********