## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing-I(4033) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any two of the following Questions: | 2 x 6 = 12 |
| --- | --- | --- |
| a) | A line PQ measuring 70mm has one of its ends 30mm in front of VP and 10mm above HP and the other end is 15mm in front of VP and is above HP. The top view of the line is 55mm long. Draw the front view of the line and find the inclination of the line with HP and VP by auxiliary method. |  |
| b) | A pentagonal lamina of sides 25mm is resting on a side on HP which is inclined at 40 to VP and the surface is inclined at 35 to HP. Draw the projection by auxiliary method. |  |
| c) | Draw the projections of a cone base 50mm diameter and axis 50mm long when it is resting on the ground on a point on its base circle with its axis making an angle of 30 with HP and 45 with VP by auxiliary plane method. |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | A cylinder of 50mm diameter and axis 100mm long is standing with its base on HP. It is completely penetrated by a horizontal cylinder of 40mm diameter and axis 80mm long such that their axis bisect each other at right angles. The axis of the penetrating cylinder is parallel to VP. Draw the projection showing curves of intersection. |  |
| b) | A vertical square prism, faces of which are equally inclined to VP base 60mm side is completely penetrated by a horizontal square prism base 40mm side so that their axis intersect. The axis of the horizontal prism is parallel to VP while the faces of the two prisms are equally inclined to HP and VP respectively. Draw the projections showing lines of intersection. |  |
| Q.No.3. | Answer any one of the following Questions: | (10) |
| a) | A vertical cylinder with base diameter of 40mm and axis 85mm has a through square hole of 15mm drilled centrally such that the axis of the hole bisects the axis of the cylinder at right angles and is perpendicular to VP. Two sides of the hole make equal inclinations to HP. Develop the lateral surface of the cylinder showing the true shape of the hole in it. |  |
| b) | Develop the lateral surface of the ‘Ý’ piece formed to connect 3 cylindrical pieces of 30mm diameter as shown in fig.(1). |  |
| Q.No.4. | Fig.(2) shows by 1st angle method of projection, the front view and top view of an object. Draw using the same angle of projection i) Top view ii) Sectional Elevation along A-A iii) Left hand side view. | (16) |
| Q.No.5. | Answer the following Questions: |  |
| a) | With the help of neat free hand sketches show the following: i) Revolved section ii) Half section iii) Full section iv) Partial section. | (8) ……2/- |
|  | -2- |  |
|  | ### OR |  |
| a) | Give conventional representation of the following: i) Concrete ii) Stud iii) Wood iv) Packing material | (8) |
| b) | Draw by double line method a 25mm diameter pipe showing: i) Gate Valve ii) Check valve iii) 90 elbow. | (9) |
| Q.No.6. | Redraw the views given in fig.(3) by proportionate free hand sketch and show the following appropriate symbols. | (8) |
| a) | Piece (1) to be welded to piece (2) by single ‘J’ butt. |  |
| b) | Piece (2) to be welded to piece (3) by all round fillet. |  |
| c) | Piece (1) to be welded to piece (4) by square butt weld finished flush on top. |  |