## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/COMP/MED/EI/E/E&EE

Subject: Digital Electronics(CS309) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following: - 25(10) = ____________(2) ii) 1001.011(2) =__________(10) - A(16) = _______(2) |  |
| b) | Convert 11001011(2) to gray code. |  |
| c) | Draw the symbol of NOR, EX-OR and ExNOR logic gates. |  |
| d) | Draw the logic diagram of T flipflop and give its Truth table. |  |
| e) | Explain Race around condition with reference to flipflops. |  |
| f) | How many flipflops are required to construct Mod20 counter? Why? |  |
| g) | Name at least two types of ADC. And list few applications of ADC. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Perform the following addition using 2’s complement method: - 125, -68 ii) -48, -23 |  |
| b) | Prove NAND gate is an universal gate. |  |
| c) | Represent the decimal numbers 396 and 4096 in - Binary number system - Hexadecimal number system - Gray code |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Simplify using K map X=ABC+ACD+AB+ABCD+ABC and draw the logic circuit for minimised expression. |  |
| b) | Explain the operation of 4 bit parallel adder with the help of a neat diagram. |  |
| c) | Differentiate between encoder and decoder. Draw neat diagram of 4 to 2 encoder using appropriate logic gates. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of Master Slave JK flipflop. Draw its symbol. What is race around condition? How can it be overcome in Master Slave JK flipflop? |  |
| b) | Draw a clocked RS flipflop and explain its working with truth table and timing diagram. |  |
| c) | With the help of a neat logic diagram and timing diagram, explain how data (101)2 is shifted into a parallel-in-serial out shift register. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of synchronous decade counter and draw the necessary diagrams. |  |
| b) | Describe how a ring counter operates, with the help of a logic diagram and timing diagrams. |  |
| c) | Write a short note on any two of the following: - Modulus of the counter - Asynchronous counter - Synchronous counter |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Draw a neat diagram of D to A converter using Binary ladder method. Also list advantages of this method. |  |
| b) | With the help of a neat diagram, describe the A/D converter which uses the successive approximation method. |  |
| c) | What is DAC? What will be the output of DAC if digital input is 110(2) and logic0=0V and Vref =10V? |  |
| d) | Draw the logic circuits for following and write its truth table. Also write its logic equations. - Half Subtractor Express the logic equation in POS form. |  |
| e) | Draw the logic diagram of 4 bit serial-in serial-out shift register, with negative edge Triggered flipflop. What is the effect on output of 4 bit SISO register, if flipflops are not negative Edge triggerable? |  |

***********