## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./EI

Subject: Circuit Theory(EE301)/(4104) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State Kirchoff’s Current law and Kinchoff’s Voltage law. |  |
| b) | A 60Hz voltage of 115 volts is impressed on 100Ω resistor. Write the equation for voltage and current. |  |
| c) | Refer to waveform diagrams of current and voltage shown in fig.1 and answer the following: - Is the p.f. leading or lagging. - What is the value of phase angle? - Which quantity is leading? |  |
| d) | The currents recorded in R, Y and B lines of 3 system under abnormal are IR=300300°A, IY=500240° and IB=100060°A. Find the zero sequence component vector. |  |
| e) | Explain single transients and double transients. |  |
| f) | State the two conditions for the AC bridge to be balanced. |  |
| g) | State i) Thevenin’s Theorem ii) Maximum Power Transfer Theorem. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the current drawn in all the branches of the network shown in fig.2. |  |
| b) | Find the current in 15Ω resistor using Thevenin’s Theorem for the network shown in fig.3. |  |
| c) | Using superposition theorem, find the current in each branch of the network shown in fig.4. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 60Hz voltage of 230V is impressed on an inductance of 0.265 Henry. Write the equation for voltage and current. Let the voltage wave start at time t=0. Show voltage and current on phasor diagram and waveform diagram. |  |
| b) | In a series circuit containing pure resistance and a pure inductance the current and voltage are expressed as i(t)=5sin314t+2π3 and v(t)=15sin314t+5π6 - What is the impedance of the circuit? - What is the value of resistance? - What is the valve of inductance? - What is the average power drawn by the circuit? - What is the value of power factor? |  |
| c) | A voltage e(t) = 100sin314t is applied to series circuit consisting of 10 ohm resistance, 0.0318 Henry inductance and a capacitor of 3.6µF. Calculate - Expression for i(t) - Phase angle between voltage and current - Power factor - Active power consumed - Peak valve of pulsating energy. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A coil having an inductance of 50mH and a resistance of 10 Ohm is connected in series with a 25µF capacitor across a 200V AC supply. Find i) Resonance frequency (ii) current at resonance (iii) value of Q. |  |
| b) | A series RLC circuit consists of R=1000Ohm, L=100mH and C=10 picoFarads. The applied voltage across the circuit is 100V. - Find the resonance frequency of the circuit. - Find quality factor of the circuit at resonance. - Find Angular frequency at half power point. - Calculate Bandwidth of the circuit. |  |
| c) | A 20 Ohm resistor is connected in series with an inductor, a capacitor and ammeter across a 25V variable frequency supply. When the frequency is 400Hz, the current is at its maximum value of 0.5A and potential difference across the capacitor is 150v. Calculate - The capacitance of the capacitor - The resistance and inductance of the inductor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A balanced star connected load of “(8+jb) Ohm per phase is connected to a balanced 3 phase 400 volts supply. Find the line current, power factor, power and total volt amperes. |  |
| b) | A 220V, 3 voltage is applied to a balanced delta connected 3 load of phase impedance (15+j20) Ohm. - find the power consumed per phase - Find the phase current and line current. - What is the phasor sum of three line currents? |  |
| c) | A star connected alternator supplies a delta connected load. The impedance of the load branch is (8+jb) ohm per phase. The line voltage is 230V. Determine (i) current in load branch (ii) Power consumed by the load (iii) Power factor of the load and (iv) Reactive power of the load. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of circuit diagram, explain the Maxwell’s bridge. The arms oif Maxwell’s AC bridge are arranged as follows. AB is the resistor of 1000 Ohm in parallel with capacitance of 0.5 micro Farads, BC is resistance of 600 Ohm and AD is the resistance if 400 Ohm. If the balance is obtained under this condition, find the value of resistance and inductance of the branch CD. |  |
| b) | Explain the Hay’s AC bridge. The four arms of Hay’s AC bridge are arranged as follows. AB is a coil of Unknown impedance, BC is resistor of 1000 Ohm, CD is a resistor of 833 OHM in series with a standard capacitor of 0.30 micro Farads, DA is resistor of 16,800 Ohm. If supply frequency is 50Hz, determine inductance and resistance at balance condition. |  |
| c) | Find out positive, negative and zero phase sequence components of the following set of unbalanced voltage vectors: VA=1030°, VB= 30-60° & VC=15145°. |  |