## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Drawing & Calculations(SH603) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Dynamic stability |  |
| b) | Subdivision and floodable length |  |
| c) | Buoyancy curve |  |
| d) | Section modulus |  |
| Q.No.2. | A vessel is loaded up ready for departure KM=11.9m, KG=9.52m with displacement of 20550t. From the ship’s cross curves of stability the GZ ordinates for displacement of 20550t and a V.C.G of 8m above base are as follows: Angle of heal (); 0; 15; 30; 45; 60; 75; 90. GZ ordinate (m); 0; 1.10; 2.22; 2.60; 2.21; 1.25; 0.36. Using the information construct the ship’s statical stability curve for condition of loading and determine the following: - Maximum righting lever - Angle of heel at which maximum lever occurs - Angle of heel at which deck just becomes immersed iv) Range of stability | (12) |
| Q.No.3. | A box shaped vessel 150m long, 20m beam is floating upright in salt water at draught of 6m F & A. The collision bulkhead is situated at 8m from forward. Find the new draft if the vessel is now bilged forward of collision bulkhead. | (12) |
| Q.No.4. | Draw and label the types of bulkheads. | (12) |
| Q.No.5. | A box shaped vessel has length 80m and breadth of 10m and is floating in light condition at a draught of 3.0m in water of relative density 1.010. It is divided into four holds of equal length. Cargo is loaded as follows: No.01 hold -120 tonnes No.02 hold - 120 tonnes No.03 hold - empty No.04 hold – 160 tonnes Construct the curve of shear force and bending moment. Calculate the maximum value stating the position where they occur. | (12) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is angle of loll and when does it occur? |  |
| b) | Why is ship considered as longitudinal beam? What are the typical values of bending moment along length of ship? |  |
| c) | Write in brief about sine correction in shift of C.G. of a ship. |  |
| d) | Draw the midship section of a bulk carrier. |  |

***********