## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(SH601)/(4339) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) Q.No.1 is compulsory. Answer any five from the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Wind resistance |  |
| b) | Geometrical similarity |  |
| c) | Propeller materials |  |
| d) | Self-propulsion test |  |
| e) | Self-propulsion feast |  |
| f) | BHP of engine |  |
| g) | Propeller efficiency |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | Explain the purpose and procedure of model testing. | (10) |
| b) | State Froude’s law of comparison. | (02) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Explain any two special propeller types with salient features, advantages and disadvantages. | (06) |
| b) | A propeller of 4.5m pitch turns at 120 rev/min and drives the ship at 15.5 Knots. If the wake fraction is 0.3, calculate the apparent slip and real slip. | (06) |
| Q.No.4. | With a neat sketch explain the working of i) Super cavitating propellers ii) Controlled pitch propellers. | (12) |
| Q.No.5. | Answer the following Questions: |  |
| a) | With neat sketch explain the construction of water-cooled stern tube. | (09) |
| b) | Discuss the need for boring stern tube in place after welding. | (03) |
| Q.No.6. | The resistance test of a vessel of length 67.25m has been conducted in a towing tank. Length of model=1.77m. Following are the results of resistance test of a model. Speed m/sec; 0.51; 0.59; 0.66; 0.75; 0.82. Resistance (RT) kg; 0.072; 0.093; 0.122; 0.16; 0.22. Determine the total resistance of a ship of length 67.25m at speed 7.5 knots. ƒmodel=1.728 SW ƒship =1.418 SW Also find the break power of engine required at 7.5 knots if propulsive coefficient is 55%. | (09) (03) |
| Q.No.7. | Answer the following Questions: |  |
| a) | How required rpm of propeller is achieved with the medium speed engine? | (04) |
| b) | Explain in brief the construction of a reverse reduction gear box. What is its function? | (08) |
|  |  | …….2/- |
|  | -2- |  |
|  | ### Only for students of revised course |  |
| Q.No.8. | Answer the following Questions: |  |
| a) | Explain the types of rudders used in shipbuilding industry with their applications. | (06) |
| b) | Explain the procedure to establish the rudder stock diameter. | (06) |

***********