## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./FT&EE

Subject: Elements of Civil Engineering(CS311)/(3003) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain with sketch, how do you measure a long straight distance (such as 200m) using a chain/tape of 30.0m length. |  |
| b) | What is meant by local attraction in compass survey? How can it be traced for a Line AB whether there exists local attraction? |  |
| c) | Differentiate between a horizontal line and a level line. |  |
| d) | Enlist the uses of a theodolite. |  |
| e) | What are the functions of foundation in Buildings? List the types of foundations. |  |
| f) | Name any 6 types of floorings for buildings. |  |
| g) | Describe the necessity and types of thermal insulations used in buildings. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with sketch, the procedure of measuring a distance along a sloping ground. |  |
| b) | Explain what is meant by whole circle bearings and Quadrantal bearings. Show three numerical examples and convert them from W.C.B to Q.B. |  |
| c) | Using a dumpy level, the following 8 consecutive readings were taken. First position of instrument: 1.550 (on a pt. of R.L. 100.00m), 1.720, 1.665, 2.005, then instrument is shifted to a new position. Second position of instrument: 0.955, 1.110, 0.850, 1.250. Enter the readings on a standard levelling format and calculate the R.L.s of all the points. Show arithmetic check also. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain stepwise the procedure of temporary adjustment of a levelling instrument at its every new setup. |  |
| b) | Explain with a neat sketch, the method of prolonging a straight line with the help of a theodolite by transitting method. |  |
| c) | Draw a neat plan and section of Raft foundation and explain where they are suitable. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw any three neat sketches of trusses used for building roofs. |  |
| b) | Write in brief what are: Floating piles, Friction piles, Cast-in-situ piles, Precast piles, Cased piles and Uncased piles. |  |
| c) | Write short notes on: Mixing, placing, compacting, curing in respect of concreting in buildings. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short notes on i) Concept of R.C.C. ii) Groutings and its uses. |  |
| b) | Draw a neat plan of a small house with following details: Total overall dimension of the house is about 6.5m x 5m. Within this rectangle:- Left portion has a living room of 4.00m x 5.00m Right portion has a kitchen of 2.5m x 3.00m and a toilet of 2.5m x 2.00m Wall thickness may be taken as 20cms Door width: 0.90m, windows width: 1.50m and 1.00m Take a scale of 1:50. |  |
| c) | Draw a neat section of the building given in 5(b) above passing through doors & windows. Take slab height as 3.00m, slab thickness: 0.12m Plinth height: 0.60m, sill height: 0.90m, Lintel level: 2.10m, doors: 2.10m height, windows: 1.20m height. Show lintels and chajjas. Take a scale of 1:50. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Compute the included angles ABC and BCD, if the bearings of the lines are: AB 120 30, BC 17830, CD26015 |  |
| b) | Explain in brief with a sketch the procedure to find horizontal angle ABC by cumulative method or repetition method accurately, using a theodolite. |  |
| c) | Draw neat sketches/symbols of any four of the following: Brick masonry in section, Stone masonry section, Wood in elevation, Concrete in section, Metal sections such as I, L, C channels. |  |
| d) | Write short notes on fire protection in buildings in respect of planning and maintenance of the building. (only for students of revised scheme) |  |
| d) | List out various sound insulating materials used in buildings and state their suitability. (only for students of old scheme) |  |

***********