## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(ME502)/(5034) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain Coefficient of Performance of Refrigeration system. |  |
| b) | Designate the following refrigerants: - Trichlorotrifluoro ethane (C2Cl3F3) - Carbondioxide |  |
| c) | Draw a neat sketch of following: - P-H diagram - T-S diagram of VCRS when the refrigerant is super heated before compression. Assume no under cooling. |  |
| d) | Define the following terms: - Relative humidity - Dew point depression |  |
| e) | With a neat sketch briefly explain any Psychrometer. |  |
| f) | What do you understand by draft? How does draft affect human comfort? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define refrigerant. Explain how the refrigerants are classified? |  |
| b) | For an ammonia based air conditioner working on vapour compression refrigeration system, temperature range is given as 295K to 261K. The gas is dry and saturated at the end of compression. Calculate the C.O.P. of the cycle. Assume no under cooling of the liquid gas. |  |
|  | Temp. K; Enthalpy KJ/Kg; Entropy KJ/KgK. Liquid; Vapour; Liquid; Vapour. 295; 61.9; 208.1; 0.197; 0.703. 261; 18.4; 234.5; 0.075; 0.896. |  |
| c) | Draw a neat sketch of Electrolux refrigerator and explain its working principle. |  |
| Q.No.3. | Write short notes with a neat sketch: (any three) | 3 x 4 = 12 |
| a) | Hermetically sealed compressor |  |
| b) | Evaporative type of condenser |  |
| c) | Dry expansion type of evaporator |  |
| d) | High-side float valve |  |
| e) | Finned tube evaporator |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | 200 m3 per minute of air at 35C dry bulb temperature and 30% RH is cooled to a temperature of 25C. Find the amount of heat added to air per minute. |  |
| b) | Describe the following process with a neat sketch: - Heating and dehumidification - Cooling and humidification |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | State the functions of components of vapour compression system. |  |
| d) | With a neat sketch explain the construction and working of Ice plant. (only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch following air distribution systems: i) Ejector system ii) Downward system. |  |
| b) | State the advantages of vapour absorption refrigeration system over vapour compression refrigeration system. |  |
| c) | Explain the following Psychrometric properties: - Saturated air ii) DBT iii) WBT iv) DPT |  |
| d) | What is the function of a filter? How are filters classified? Explain any one filter with a neat sketch. (only for students of old scheme) |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Room sensible heat factor |  |
| b) | Automatic humidity control |  |
| c) | Duct design (any method) |  |
| d) | Requirements of good room air distribution |  |
| e) | Automatic temperature control |  |
| f) | Limit switch (only for students of old scheme) |  |
| g) | Application of cold storage (only for students of old scheme) |  |

***********