## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(EE601)/(4115) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw the typical block diagram of a closed loop drive system and list its various components. |  |
| b) | State any three points of comparison between individual drive and group drive. |  |
| c) | Briefly explain the principle of v/f method of speed control of 3 Induction motor. State one of its advantages. |  |
| d) | List any six types of enclosures for motor as per IS 4722. |  |
| e) | Draw and briefly explain the typical heating and cooling curve of a drive motor. |  |
| f) | State the type of drive motor used for the application of lift. Justify the answer. |  |
| g) | Name your choice of drive motor for driving pump load. Justify with reasons. |  |
| h) | List the various types of drive motors used in a paper industry. (Only for the students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the four quadrantal operation of a drive motor for passenger lift load. Draw the relevant diagram. |  |
| b) | Draw and explain in brief the joint speed torque characteristics of a motor-load combination. State its signification. |  |
| c) | With the help of neat diagram explain the determination of referred moment of inertia for a motor load combination. Write the relevant equation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define braking. State the advantages and limitations of electric braking. |  |
| b) | List the methods of electric braking applicable for use with DC series motor. Explain any one of these methods with circuit diagram and characteristics. |  |
| c) | With neat diagram and characteristics explain plugging in case of 3 Induction motor. State any one of its limitations. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the various pole changing methods of speed control of 3  induction motor. With neat diagram explain any one of these methods. |  |
| b) | On the basis of heating, select a suitable power rating for drive motor and draw the bar graph (load v/s time) for the following intermittent duty: - Constant load of 35 KW for 3 seconds - Constant load of 15 KW for 20 seconds - Constant load of 35 KW for 2 seconds - Constant load of 10 KW for 15 seconds Between the operating periods 2) & 3) above, there is a rest period for 37 seconds followed by another rest period of 43 seconds after 4). The duty cycle repeats indefinitely. |  |
|  |  | ……2/- |
|  | -2- |  |
| c) | With neat diagrams of the heating & cooling curves briefly explain any four classes of duty cycle of motors. State suitable example of each duty cycle. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the need of interlocking operation of contactors. Draw the control and power circuit diagram for indirect reversal of 3 induction motor rotation using interlocking operation. |  |
| b) | Draw the control and power circuit diagram for remote control operation of 3 induction motor and briefly explain the same. |  |
| c) | Briefly explain the working and one application of each of the following: i) Contactor ii) Time delay relay iii) Float switch. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the selection of drive motor for the following applications and justify the same with reasons: i) Lathe ii) Refrigeration & Air conditioning iii) Machine tools |  |
| b) | With neat circuit diagram explain the principle and procedure for static rotor resistance control method for speed control of 3 induction motor. |  |
| c) | With neat control diagram explain the sequential operation of three motors A, B & C such that motor A starts first followed by B and then C. (Use sequence control using TDR) |  |

***********