## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(CE405)/(4025) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Density ii) Specific gravity iii) Viscosity. |  |
| b) | Distinguish between an orifice and a mouthpiece. |  |
| c) | Explain total pressure and centre of pressure. |  |
| d) | Explain water hammer effect in pipes. |  |
| e) | What is priming of a centrifugal pump? |  |
| f) | What is a weir? Give two differences between weir & notch. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A pipe 300m long has a slope of 1 in 100 and tapers from 1m diameter at a higher end to 0.5m at lower end. The quantity of water flowing is 900 litres/second. If the pressure at the higher end is 70KPa, find pressure at the lower end. |  |
| b) | An isosceles triangular plate 3m high and 3m base is immersed vertically in water in such a way that its apex is below the base and at a distance of 3m below the water surface. Find total pressure and centre of pressure. |  |
| c) | With the help of a neat sketch explain construction and working of Bourdon’s Tube Pressure Gauge. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the most economical cross section of a rectangular channel to carry 0.3 m3/s of water, when bed slope is 1 in 1000. Assume chezy’s constant C=60. |  |
| b) | A trapezoidal channel with 3m base and side slopes 1:1 carries water with a depth of 1m. The bed slope is 1 in 1600. Estimate the discharge. Take value of N in Manning’s formula as 0.04. |  |
| c) | With the help of a neat sketch explain working principle of reciprocating pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A circular plate of 2m diameter is immersed vertically in water such that its centre is 2.5m below the water level. Calculate the total pressure on the plate and centre of pressure from free surface. |  |
| b) | A venturimeter is connected to a horizontal pipe of 500mm. The diameter of its throat is 250mm. Calculate the discharge of water in litres per second through the pipe, if pressure difference between the inlet and throat of venturimeter is 4m of water. Meter coefficient is 0.97. |  |
| c) | A jet of water is coming out of a sharp edged vertical orifice under a constant head of 10cm. At a certain point of the jet, the horizontal and vertical co-ordinates from vena contracta are 40cm and 42cm respectively. Determine i) Coefficient of velocity ii) Coefficient of contraction if Coefficient of discharge is 0.62. |  |
|  |  | ……..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A compound pipeline 1650m long is made up of pipes of 450mm diameter for 900m, 375mm for 450m and 300mm for 300m. If the compound pipe is to be replaced by a pipe of uniform diameter, find diameter of the new pipe, assuming the length to remain the same. |  |
| b) | Water flows over a rectangular notch of 1m length over a depth of 150mm. Then the same quantity of water passes through a triangular right angled notch. Find the depth of water through the notch. Take the coefficients of discharges for the rectangular and triangular notch as 0.62 and 0.59 respectively. |  |
| c) | Write short note on i) Hydraulic jump ii) Water hammer. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain working of piezometer with the help of a neat sketch. |  |
| b) | Write a short note on i) Borda’s mouthpiece ii) Minor losses in pipes. |  |
| c) | Explain the working principle of a centrifugal pump. |  |
| d) | Explain wetted perimeter, hydraulic mean depth in case of rectangular & trapezoidal channel. |  |

***********