## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED/E&EE

Subject: Microprocessors(8085)-I[4142] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Name the buses used in the 8085 based system and briefly explain their use. |  |
| b) | State and explain the operation performed by the following program, MVIA,02H RLC HLT |  |
| c) | Name and state the functions of the control signals sent out by the 8085 for memory and I/O. Draw a neat diagram to show how they are generated. |  |
| d) | Define the following: i) Instruction cycle ii) Machine cycle (iii) T states |  |
| e) | Define stack. What is the use of the stack pointer register? |  |
| f) | List the different interrupts of the microprocessors 8085 as per their priorities and explain the one which has the highest priority in brief. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State & explain the internal data operation of the 8085 based microprocessor system. |  |
| b) | What is the use of multiplexing the address bus? With help of a diagram explain why the lower order address bus has to be demultiplexed in a 8085 system. |  |
| c) | In a microprocessor system state the functions of: (i) Accumulator (ii) Program counter (iii) All the flags |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify instructions according to the size of the byte and give the least 4 examples of each. |  |
| b) | List and explain the addressing modes of the instruction in an 8085 based system and give at least two example of each. |  |
| c) | Draw and explain the timing diagram of the instruction MVI A, 32H. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a flow chart and with a program to add 10 numbers stored in memory location 20C0H onwards. Store the sum in the location 20D0H. |  |
| b) | Write a program to interchange two numbers stored in memory location 20D0H and 20D1H. Draw a flowchart. |  |
| c) | Draw a flow chart and write a delay routine using a register pair. Calculate the delay generated if the register pair of load with FFFFH for an 8085 Microprocessor system. |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the following instruction: i) LHLD 2050H ii) XCHG iii) ADC M iv) SP HL |  |
| b) | Show with a diagram and necessary instructions, why stack is called last in first out (LIFO). |  |
| c) | State the number of machine cycles and T states of a CALL instruction. Explain the various operations performed during each of the machine cycles of the CALL instruction. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of multiple interrupts and priorities in the 8085 microprocessor. |  |
| b) | Explain the SIM and RIM instruction of the 8085 microprocessor. |  |
| c) | With the help of a diagram explain how an input device can be interfaced to the microprocessor. |  |

***********