## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Engineering-II(SH402) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Crankcase explosion relief valve |  |
| b) | Oil separator in a refrigeration plant |  |
| c) | Temperatures maintained in Fish room, meat room and vegetable room |  |
| d) | Types of drives for deck machinery |  |
| e) | Principle of variable delivery pump in a hydraulic steering |  |
| f) | Statutory requirements for steering gears of ocean going ships |  |
| g) | Interlock for shore supply of power to a ship |  |
| Q.No.2. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Briefly describe the telemotor system for hydraulic steering with simple sketch. |  |
| b) | With a simple line sketch, show the arrangement of an emergency switchboard on a ship. |  |
| Q.No.3. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Show the location of various deck machinery on the plan /elevation of a ship. Briefly describe the function of anchor windlass. |  |
| b) | Describe the functions of a cargo handling crane. What safety devices are fitted to avoid damage/accident? |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | Make a list of the major components of a 2 stroke slow speed diesel engine with uniflow scavenging. Name any three safety devices fitted on such engines. |  |
| b) | Sketch and briefly describe the lubricating oil system of an ocean going ship fitted with a slow speed engine. |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | Sketch and describe the timing diagram of a four stroke engine clearly showing the timing of opening and closing of valves and fuel injection in different strokes. |  |
| b) | Draw a vapour compression refrigeration system and briefly describe the function of each constituent. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain the conditions for switch on an incoming generator (AC) for parallel operation. |  |
| b) | Give the list of mooring equipment on a ship. |  |
| c) | What parameters are controlled in air conditioning? |  |
| d) | What is Tappet clearance? Why is it necessary? |  |
| e) | Write briefly on types of bearings used in diesel engines. |  |

***********