## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms “Draw bar pull” & “Rolling Resistance’’. |  |
| b) | What are the functions of “Leaf spring” & “coil spring” in a vehicle suspension system? |  |
| c) | How is stability of vehicles on slopes and turns achieved? |  |
| d) | Distinguish between a “Limousine” and “Saloon or Sedan” car body styles. |  |
| e) | What is hot dip galvanization? |  |
| f) | Explain the necessity of “Toe-in, Toe-out”. |  |
| g) | Why is a band brake used in Automatic Gear box? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A motor vehicle of total weight 12000N has four road wheels of 0.61m effective diameter. The effective moment of inertia of the four road wheels and rear axle together is 63.25 N-m2 while that of the engine and flywheel is 6.325N-m2. The transmission efficiency is 90% and tractor resistance at a speed of 24 Km/hr is 226.8N. The total available engine torque is 207.5 N-m. Determine the gear ratio, engine to back axle to provide maximum acceleration. |  |
| b) | With empirical relations explain the types of resistances influencing the performance of a vehicle. |  |
| c) | Explain in detail vehicle vibrations like rolling, bouncing and swaying. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you mean by “instantaneous centre” in steering geometry? Sketch and explain Ackerman steering gear mechanism. |  |
| b) | Explain with a neat sketch rack and pinion steering mechanism. |  |
| c) | What do you mean by front wheel alignment? Explain in detail with sketch all the angles to be set in achieving it. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a simple sketch, explain construction & working of torque convertor. |  |
| b) | Sketch and explain a hydramatic belt driven type gear box. |  |
| c) | Sketch and explain the working of an automatic transmission. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain swinging half axle type independent suspension. |  |
| b) | With a neat sketch explain the construction and working of Macpherson strut type of independent suspension. |  |
| c) | With the help of a neat sketch, explain the construction & working of a Telescopic shock absorber. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Aerodynamic coefficient? Why is stream lining of an automobile body required? |  |
| b) | With a neat sketch explain the driver seat mechanism of a car. |  |
| c) | How is protection of a Automobile body done by metallic, organic and in organic coatings? |  |

***********