## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(CE504)/(4026) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define a Flitched beam and Transformed section. |  |
| b) | State the middle third rule. |  |
| c) | Define Principal planes and Principal stresses. |  |
| d) | State the advantages and disadvantages of Fixed beam. |  |
| e) | Define Relative stiffness and Distribution factor. |  |
| f) | Define effective length and Slenderness ratio. |  |
| g) | State the expressions for determination of maximum slope and deflection in a cantilever beam and simply supported beam carrying UDL throughout. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Flitched beam consists of two wooden beams of 150x300mm in size with a steel plate of 12mm thick and 200mm deep placed symmetrically between wooden beams. If beam is simply supported for 6m span with UDL of 16KN/m throughout, determine stresses in steel and wood. Take Es=15Ew |  |
| b) | A rectangular column of 300x450mm carries a vertical load of 600KN at an eccentricity of 120mm in a plane bisecting the depth of 450mm side. Find maximum and minimum combined stresses. |  |
| c) | A rectangular column of 250x450mm in size has a cavity of 100mm diameter inside symmetrically. Find the magnitude of load to be placed on XX axis at an eccentricity of 75mm so that resultant maximum stress is 65N/mm2. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The normal stresses at a point in a steel bar are 200MPa (tensile) and 100MPa (compressive). Determine the normal, tangential and resultant stresses on a plane inclined at 50 to the axis of major normal stress. Also find magnitude of maximum shear stress. |  |
| b) | A point in a beam part is subjected to tensile stress of 125N/mm2 and compressive stress of 80N/mm2 on two mutually perpendicular planes with a shear stress of 50N/mm2 across both planes. Find magnitude and direction of principal stresses and maximum shear stress. |  |
| c) | A simply supported beam of 4m span and 200x250mm in section carries a UDL of 25KN/m throughout span. Find maximum slope and deflection in beam. E=0.15x105 MPa |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | A simply supported beam of 6m span carries point loads of 25KN, 40KN and 15KN at a distance of 2m, 3m and 4m from left support. Using Macaulay’s method determine deflection under all the point loads. EI=15x1012 N-mm2 | (10) |
|  | ### OR |  |
| a) | A fixed beam of 5m span carries point loads of 20KN, 40KN and 10KN at a distance of 2m, 3m and 4m from left support. Analyse the beam and sketch SFD and BMD. | (10) …..2/- |
|  | -2- |  |
| b) | A fixed beam of 4m span carries UDL of 30KN/m throughout. Find end moments and sketch FEM diagram. | (2) |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A continuous beam ABC has fixed support at A and simple support at B and C. The span length of AB and BC are 6m and 4m respectively. Span AB carries UDL of 40KN/m throughout while BC carries a point load of 30KN at 1m from end C. Flexural rigidity of AB is 2EI and that of BC is 1.5EI. Using moment distribution method, analyse the beam and sketch SFD and BMD of beam. |  |
| b) | A continuous beam ABCD has simple supports at A, B, C and D. The span lengths AB=6m, BC=5m and CD=4m. Load on AB span is 40KN/m throughout, BC span UDL of 20KN/m and CD span UDL of 30KN/m throughout. EI is uniform. Analyse the beam by moment distribution method and sketch SFD and BMD of beam. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | State the assumptions made in Euler’s theory of long column and its limitations. | (4) |
| b) | A built up column consists of four angle sections of size 150x150x12mm placed at corners of overall box size of 300x300mm size symmetrically. Effective height of column is 6m. Determine safe load carried by column with a factor of safety as 2.5. Properties of each angle section are Ixx=Iyy=735.4x104mm4, A= 3459mm2, Cxx= Cyy=41.4mm Take E = 2x105 N/mm2 | (8) |
|  | ### OR |  |
| b) | A built up column consists of two channel sections ISMC 400 placed with its web facing each other of box section of overall dimension 200x400mm. Effective height of column is 8m. Using Rankine’s formula, find Buckling load of column. Given, Rankine’s constant =1/7500 and crushing stress=300MPa. Properties of each channel section: A = 6291 mm2, Ixx = 15082x104mm4, Iyy = 504x104mm4, Width of flange = 100mm and Cyy = 24.2mm | (8) |

***********