## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electrical Engg.

Subject: Generation & Transmission(4114) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define i) Diversity factor ii) Demand factor. |  |
| b) | State any 3 factors to be considered while deciding the number of units for a power plant and prepare the operating schedule. |  |
| c) | State the specifications for procurement of standby Diesel generator set. |  |
| d) | Explain in brief any one cooling methods used in standby diesel generator sets. |  |
| e) | What do you mean by skin effect in transmission line? |  |
| f) | Define Voltage Regulation & Transmission Efficiency. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State 3 merits and 3 demerits of interconnected grid system. |  |
| b) | With a neat diagram, explain generation of power in hydro-electric power plant. |  |
| c) | Write a short note on wind power generation. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain main components of Diesel Generator set & their functions. |  |
| b) | What are Bundled conductors? State any 3 advantages of using Bundled conductors. |  |
| c) | State any 6 points of differences between pin & suspension insulators. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the main components of HVDC transmission system. |  |
| b) | Explain the effect of variation of transmission voltage & load power factor on performance of short transmission line. |  |
| c) | A 1 overhead transmission line delivers 1100KW at 33KV at 0.8 p.f. lagging. The total resistance & inductive reactance of the line are 10Ω & 15Ω respectively. Determine i) Sending end voltage ii) Sending end power factor iii) Transmission efficiency. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is Corona? Explain methods to reduce effect of corona. |  |
| b) | A 1 medium transmission line 100Km long delivering 15000KW at 0.8 power factor lag has following constants: Resistance/Km = 0.25Ω, Reactance/Km = 0.8Ω, Susceptance/Km= 14x10-6 siemen, Receiving end line voltage = 66,000V Using end condenser method determine: i) Sending end current ii) sending end voltage iii) Regulation. |  |
| c) | Explain in brief various supporting structures used in overhead transmission lines. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief methods employed to reduce vibrations in overhead transmission lines. |  |
| b) | Write a short note on stringing of conductors. |  |
| c) | i) Write a short note on sag template. ii) Explain in brief any one method of tower erection. |  |

***********