## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/MED/EI

Subject: Analog Electronics(EN405) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List the Ideal characteristics of op Amp. |  |
| b) | Draw the equivalent circuit of Op Amp. |  |
| c) | Draw the block diagram of a voltage series feedback amplifier. Write the expression for voltage gain. |  |
| d) | Draw the circuit of voltage comparator with positive reference. Draw the output waveforms. |  |
| e) | Draw the frequency response of i) LPF ii) HPF. |  |
| f) | Draw the circuit of Astable Mutivibrator using IC555. |  |
| g) | State the merits of negative feedback. |  |
| h) | Define the following parameters of a regulator: i) Load Regulation ii) Line Regulation iii) Ripple Rejection |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat diagram the circuit of a Dual Input Balanced Output Differential Amplifier. |  |
| b) | Draw and explain the block diagram of OP Amp. |  |
| c) | Define the following OP Amp parameters: - CMRR (iv) Input offset voltage - Slew Rate (v) Input Resistance - Bandwidth (vi)Output Resistance |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of voltage shunt feedback amplifier. Write the expression for voltage gain of the amplifier. |  |
| b) | With the help of a circuit diagram, explain opamp as a Summing and Averaging amplifier. |  |
| c) | Explain voltage to current convertor with the help of a neat diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit of Differentiator using Op Amp. Derive the equation for output voltage and draw the waveforms for Sine, Square and Triangular waveforms. |  |
| b) | With a neat diagram, explain the Peak Detector using Op Amp. |  |
| c) | Draw the circuit of first order Butterworth High Pass filter. Write the expression for voltage gain magnitude. Write the values for: VoVin for f  fL VoVin for f = fH VoVin for f > fL |  |
| d) | With a neat diagram explain the working of Astable Multivibrator using OPAMP. | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the working of Monostable Multivibrator. |  |
| b) | Draw the circuit of Wein Bridge Oscillator. Design the same for fo = 1.5KHz with a gain of 3. |  |
| c) | Draw the circuit of first order Butterworth LPF for a cutoff frequency of 2 KHz and a pass band gain of 1. |  |
| d) | Explain with a circuit diagram the working of an Opamp series voltage regulator. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | IC 723 as a High Voltage Regulator |  |
| b) | Triangular Wave Generator using Op Amp |  |
| c) | Block diagram of IC555 |  |
| d) | Three pin Fixed Voltage Regulator |  |
| e) | Schmitt Trigger Circuit |  |

***********