## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(CE301)/(4001) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write the detailed classification of survey. |  |
| b) | Explain the terms: i) Triangulation ii) Baseline iii) Offset. |  |
| c) | Explain general principles of survey. |  |
| d) | Define levelling and state its purpose. |  |
| e) | What is parallax? How it can be eliminated? |  |
| f) | What are the merits of Plane table survey over the other methods? (only for students of revised scheme) |  |
| f) | Explain the use of Abney level. (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how will you continue chaining past the following obstacles: - River ii) Tall building |  |
| b) | Explain chain and cross–staff survey for locating ground features with neat sketch. |  |
| c) | Explain with neat sketch construction & use of Prismatic compass. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the methods of chaining on slopping ground. |  |
| b) | What are the sources of errors in compass survey? What precautions will you take to eliminate them? |  |
| c) | Determine the values of Included angles in a closed compass traverse ABCD conducted in the clockwise direction given the following FB of their respective lines. Line FB AB 40 BC 70 CD 210 DA 280 Apply the necessary check. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following Fore & Back bearings were observed while traversing an area with a compass. Line FB BB PQ S 37 30 E N 37 30 W QR S 43 15 W N 44 15 E RS N 73 00 W S 72 15 E ST N 12 45 E S 13 15 W TP N 60 00 E S 59 15 W Find the corrected bearing of the line. |  |
| b) | Explain permanent and temporary adjustments of a dumpy level. |  |
|  |  | …….2/- |
|  | -2- |  |
| c) | The following consecutive readings are taken with level & levelling staff on continuously sloping ground at common intervals of 30m: 0.602, 1.234, 1.860, 2.574, 0.238, 0.914, 1.936, 2.872, 0.568, 1.824, 2.722. The RL of the first point was 198.75m. Rule out the page of level field book & enter the above readings & calculate the R.L.s of all the points. Apply necessary checks. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain simple levelling & compound levelling with neat sketches. |  |
| b) | Describe HI method of computing levels. State the advantages & disadvantages of HI method & rise & fall method. |  |
| c) | What are the conventional signs used to represent the following: i) Embankment ii) Railway line iii) River iv) Marshy land. |  |
| d) | Describe orientation of Plane table? What are the different methods for orienting plane table? Explain any one with neat sketch. (only for students of revised scheme) |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write short note on ‘Open and Closed traverse survey’. |  |
| b) | Give the classification of leveling. |  |
| c) | Explain Direct & Indirect ranging. |  |
| d) | Define: i) Change point ii) Fore bearing & Back bearing iii) Back sight iv) Height of instrument. |  |
| e) | Write short note on ‘Plani meter’. (only for students of old scheme) |  |

***********