## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Engineering Technology

Subject: Applied Physics-I(GN103) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: | 10x1.5=15 |
| --- | --- | --- |
| a) | Define Derived Quantity. |  |
| b) | Define Sliding Vector. |  |
| c) | Give two points of distinction between distance and displacement. |  |
| d) | Why is steel more elastic than rubber? Explain. |  |
| e) | State Stoke’s Law. |  |
| f) | Define Radiation. |  |
| g) | A gas at 13C in heated at constant pressure till its volume is doubled. What is its final temperature? |  |
| h) | Define 1 coulomb. |  |
| i) | Define absolute potential at a point. |  |
| j) | Define angle of repose. (only for Mechanical Engg. & allied branches) |  |
| j) | Draw a neat diagram of energy band for semiconductor. (only for Electronics Engg. & allied) |  |
| Q.No.2. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Write the dimensional equation for the following: - Surface tension (ii) Work |  |
| b) | Check the correctness of the following equation: E = mc2 where E is energy, m is mass and c is velocity of light. |  |
| c) | Draw the graph for uniform retardation and show that the area under the curve is equal to the distance covered. |  |
| d) | A train starts from rest and accelerates at a rate of 1.1 m/s2. Calculate the velocity in Km/hr at the end of 375 m. |  |
| e) | A feather is dropped on the moon from a height of 1.5 m. The acceleration of gravity on the moon is 1.67 m/s2, find the time of fall on surface of the moon. |  |
| f) | What is collinear vector and coplanar vector? |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Define Rigidity modulus and obtain an expression for it. |  |
| b) | A wire of length 2 m is fixed at one end and a force of 10N is applied at the other end. The area of cross section of wire is 2 x 10-6 m2 and the young’s modulus of elasticity of its material is 2x1011 N/m2. Calculate the stress & strain. |  |
| c) | Define angle of contact & draw the diagram in case of liquid which wet the wall of the container. |  |
| d) | Explain determination of surface tension of a liquid using travelling microscope. |  |
| e) | Explain Terminal velocity & state Newton’s law of viscocity. |  |
| f) | Find the velocity of flow of liquid of viscocity 0.02Ns/m2 and density 1.2gm/cm3 through a pipe of radius 0.05m & Reynold’s number = 2000. |  |
| Q.No.4. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | State the law of thermal conductivity and obtain the expression for the quantity of heat flowing at steady temperature. | …….2/- |
|  | -2- |  |
| b) | Derive gas equation PV=RT. |  |
| c) | A gas at 15C is heated at constant pressure till its volume is doubled. What is its final temperature? |  |
| d) | A certain mass of gas occupies 40cm3 at 27C and 380mm of pressure. Find its volume at 47C and 680mm of pressure. |  |
| e) | Define potential difference between two points and also define absolute potential at a point. |  |
| f) | Derive the expression for two capacitors connected in parallel. |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | State coulomb’s law of electrostatics and write the expression for force and explain each term in the expression. |  |
| b) | Define 1 Farad and 1 Volt. |  |
| c) | Two capacitors of 0.6F and 1.4F are connected in parallel and then charged to 250µc. Find the capacitance of the combination and charge in each capacitance. |  |
| d) | Calculate the potential at a point 15cm from a charge of 2.75 x 10-9 coulomb in a medium of dielectric constant 2.2. |  |
| e) | State Biot Savart’s Law and write the expression for magnetic field at a point due to a current carrying conductor and state each term in the expression. |  |
| f) | Explain magnetic effect of current demonstrated by oersted’s experiment. |  |
|  | Only for Mechanical and Allied branches: |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Define a system of concurrent forces and collinear forces. |  |
| b) | What is Static friction and Dynamic friction? |  |
| c) | Write four applications of friction. |  |
| d) | A body of weight 50Kg resting on a horizontal plane is moved by applying a force of 15Kg inclined at an angle of 20 with the horizontal. Find the coefficient of friction. |  |
| e) | Define angle of repose and show that tan = µ. |  |
| f) | The algebraic sum of two forces is 25N. Their resultant which is perpendicular to the smaller force is a force of 15N. Find the magnitude of the two components. |  |
|  | Only for Electronics & Allied branches: |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Draw Energy band structure of conductor, semiconductor and Insulator. |  |
| b) | Write three differences between N-type and P-type semiconductor. |  |
| c) | With the help of diagram explain about depletion region. |  |
| d) | Define semiconductor. What types of impurities are added to semiconductor such as P-type & N-type? |  |
| e) | With the help of neat diagram explain the process of P-type semiconductor. |  |

***********