## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/EI/COMP/E/E&EE/MED

Subject: Digital Electronics-I(4134) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following into decimal: - 1101101.112 - 18F16 |  |
| b) | Explain De Morgan’s Laws. |  |
| c) | Implement the following gates using NAND gates only: - AND gate - OR gate |  |
| d) | Define the following for a flip flop: - Hold time - Setup time - Propagation delay |  |
| e) | Draw a logic diagram of a 4-bit asynchronous down counter. |  |
| f) | List one main advantage of each of the following logic families: - I2L (ii) ECL (iii) CMOS |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Perform the following using 2’S complement arithmetic: - 8010 + (- 45)10 - 9310 - 1610 |  |
| b) | Simplify the following expression and draw the NOR – NOR circuit for the simplified expression: Y (A,B,C,D) =  M (0, 1, 2, 3, 4, 5, 8, 9, 10, 11) |  |
| c) | Verify the following: - (A+B) (AB+C) +AB = A+B - A [ B+C (AB+AC) ] = AB |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the logic diagram of a clocked RS FF using NAND gates. Write its truth table and explain its working. |  |
| b) | Construct a clocked JK flip flop from a basic RS flip flop. Explain its working for different JK inputs. |  |
| c) | With a logic diagram explain how a JK Master Slave FF overcomes the race condition in a JK FF. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a logic diagram how data 012 is shifted in and out of a SISO register. |  |
| b) | Draw the logic diagram of a 3-bit Johnson counter. Draw the output waveforms. Assume the counter is initially cleared. |  |
| c) | Draw the logic diagram of a 2-bit synchronous up-down counter. Explain how it operates as an up counter. Draw the output waveforms for the up counter. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Draw a mod 5 counter that progresses through the states 000, 001, 010, 011, 100. Draw the output waveforms. | ……..2/- |
|  | -2- |  |
| b) | Draw the circuit for the following: - n-channel MOS 2 input NAND gate - n-channel MOS 2 input NOR gate |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the TTL gate with totem-pole output. |  |
| b) | Draw the basic circuit of the HTL family. Explain why the circuit has a high threshold to noise. |  |
| c) | Draw the circuit of a 2 input CMOS NAND gate. Explain how it functions as a NAND gate. |  |

***********