## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Electronics Engg./EC/COMP/EI

Subject:Basic Electrical Engg.(CS304)/Elements of Electrical Engg.(3001)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give comparison between core type and shell type transformers. |  |
| b) | Which are the different methods of reducing earth resistance? |  |
| c) | Explain the different losses in transformers. |  |
| d) | Why ‘starters’ are necessary in DC motors? |  |
| e) | Explain the construction of squirrel cage rotor. |  |
|  | Only for students of old scheme: |  |
| f) | State any three points to be considered before selecting a particular type of wiring. |  |
| g) | Mention advantages and disadvantages of cleat wiring. |  |
|  | Only for students of revised scheme: |  |
| f) | Define i) Average value ii) RMS value iii) Form factor. |  |
| g) | Define i) Real power ii) Reactive power iii) Apparent power. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on ‘Efficiency of transformers’. |  |
| b) | Write a short note on ‘Three phase AC distribution system’. |  |
| c) | With the help of circuit diagram explain the method of reversal of direction of rotation of DC series motor and DC shunt motor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain the construction of DC motor. |  |
| b) | Explain the working principle of DC motor. |  |
| c) | Explain the necessity of starter for starting induction motor. |  |
| d) | A house is fitted with 10 lamps of 100 watts each working for 5 hours a day, 1 electric geyser of 2000 watts for 2 hours a day, 10 ceiling fans of 100 watts each working for 8 hours a day and 1 electric press of 220 Volts drawing 2 amps working for 2 hours a day. Calculate the electricity bill for a month of 30 days at the rate of Rs.1.50 per unit. (only for students of old scheme) |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working principle of a single phase transformer. |  |
| b) | Explain the working principle of three phase Induction motor. |  |
| c) | What is Earthing? Explain the necessity of earthing. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat diagram, explain the construction of rewirable fuse (Kitkat fuse). |  |
| b) | Give comparison between squirrel cage induction motor and slip ring induction motor. |  |
| c) | Explain the principle of operation of ELCB. | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on MCB. |  |
| b) | i) Explain the advantage of three phase four wire system. ii) State the applications of three phase four wire system. | (4) (2) |
| c) | i) Define 1) Equipment earthing 2) System earthing. ii) State any four IE rules relevant to earthing. | (2) (4) |

***********