## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(AT603) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the term ‘aerodynamic lift’. |  |
| b) | Define toe-in and explain why it is maintained. |  |
| c) | State the functions of a suspension system in an automobile. |  |
| d) | Distinguish between leaf spring and coil spring. |  |
| e) | Draw a neat sketch of an epicyclic gear set. Show the arrangements for different speed ratios in a tabular form. |  |
| f) | Distinguish between a manual crash type gearbox and an automatic transmission. |  |
| g) | List the desirable properties of an automotive tyre. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, show the various parts of a car body. |  |
| b) | Explain induced drag. |  |
| c) | With the help of a neat sketch, explain industrial automotive painting process. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Prepare a trouble shooting chart for any one of the following faults: i) Vehicle wander ii) Wheel wobble. |  |
| b) | With the help of a neat sketch, explain the construction and working of a rack and pinion type of steering gearbox. |  |
| c) | Explain integral type of hydraulic power steering. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a torsion bar. |  |
| b) | With the help of a neat sketch, explain the working of a twin-tube shock-absorber. |  |
| c) | With the help of a neat sketch, explain ‘trailing arm suspension’ system. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of hydraulic automatic transmission. |  |
| b) | Explain the working of continuously variable transmission. |  |
| c) | Explain the working of any one of the following in an automatic transmission vehicle: i) Fluid flywheel ii) Torque converter. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the construction of a tubed tyre. |  |
| b) | List the advantages of tubeless tyre over the tubed type. |  |
| c) | With the help of a neat sketch, explain the importance and the procedure/sequence of tyre rotation for heavy vehicles. |  |

***********