## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Civil Engg.

Subject: Soil Mechanics(CE505) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the geological origin of soil with reference to soil profiles in India. |  |
| b) | Explain the effect of particle size and shape on engineering properties of soil. |  |
| c) | Enlist the factors contributing to shear strength of soil. |  |
| d) | Define ultimate bearing capacity, safe bearing capacity and allowable bearing pressure. |  |
| e) | Define total stress, effective stress and neutral stress. |  |
| f) | Explain the influence of soil conditions on exploratory programme. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define void ratio and porosity and derive the relation between them. |  |
| b) | A soil sample has a porosity of 40%. The specific gravity of solids is 2.70. Calculate i) void ratio ii) Dry density iii) Unit weight of soil if soil is 50% saturated and iv) Unit weight if soil is completely saturated. |  |
| c) | Explain Atterberg’s limits of consistency and state the practical significance. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain gradation of soil particles and its influence on engineering properties. |  |
| b) | With aid of neat sketch explain constant head permeability test. |  |
| c) | Briefly explain shear failures in soils with examples. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain settlement due to construction operation and lowering of water table. |  |
| b) | Briefly explain triaxial shear test and compare it with direct shear test. |  |
| c) | Explain plate load test and its limitations. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How will you find field density of soil by sand replacement method? Explain. |  |
| b) | Explain the factors affecting compaction. |  |
| c) | Briefly explain dilatometer test. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Soil sampling |  |
| b) | Seepage velocity and coefficient of permeability |  |
| c) | Factors affecting permeability |  |
| d) | Soil Nailing |  |
| e) | Soil Stabilisation |  |
| f) | Soil Grouting |  |

***********