## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain Gay-Lussac’s Law. |  |
| b) | What is dryness fraction of steam? What is its value in case of dry steam? |  |
| c) | Define the term Calorific Value. What is the difference between Lower Calorific Value & Higher Calorific Value? |  |
| d) | State any six applications of compressed air. |  |
| e) | Draw a labelled sketch of Air Preheater. |  |
| f) | Represent Carnot Cycle on PV diagram & write its efficiency equation. |  |
| g) | State any three advantages of Solar energy. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the use of Boys Gas Calorimeter. |  |
| b) | The percentage composition of flue gases is CO2 16%, CO 1%, O2 7% and N2 76%. Convert this analysis on mass basis to percentage volumetric analysis. |  |
| c) | Write short note on (i) Geothermal Energy (ii) Biogas. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and prove the Characteristics Gas Equation. |  |
| b) | 1.2m3 of air at a pressure of 8 bar (absolute) and temperature 70C is expanded isothermally until the volume becomes 6m3. Calculate (i) Final Pressure (ii) Work done (iii) Change in Internal Energy. |  |
| c) | Explain the following gas processes using a P-V and T-S plots: (i) Isobaric process (ii) Isentropic Process. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the Internal energy of 1 Kg of steam at - 12.1 bar pressure and 225C - 12.1 bar pressure and dryness fraction 0.9 |  |
| b) | Explain with the help of PV and TS diagram Modified Rankine Cycle. |  |
| c) | With a neat sketch explain any one type of Steam Calorimeter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe with a neat sketch construction and working of Babcock and Wilcox Boiler. |  |
| b) | Explain the construction and working of Rotary Air Compressor. |  |
| c) | With a neat sketch explain construction and working of Feed Check Valve. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Compressed Air Motor |  |
| b) | Steam Injector |  |
| c) | Intercooling |  |
| d) | Lanchashire Boiler |  |

***********