## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Architectural Engg.

Subject: Building Construction-IV(AE402) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List any four types of trusses in steel. |  |
| b) | What is integral damp-proofing? |  |
| c) | What is the difference between waterproofing of terraces & overhangs? |  |
| d) | What is the advantage of using pointing in wall finishes? |  |
| e) | List any three components of a timber floor. |  |
| f) | List the various types of weld connections. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch & write short notes on: i) Bolt connection ii) Rivetting. |  |
| b) | Explain with a neat labelled sketch, Stanchion & Beam connection. |  |
| c) | Draw a neat labelled elevation of a King Post steel truss for a span of 3m. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat labelled elevational diagram of any 2 types of Stanchion. |  |
| b) | Write short notes on: i) Tubular steel truss ii) North light truss. |  |
| c) | List any 4 principles of framing & spanning a steel structural system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write any four characteristics of an ideal damp-proofing material. |  |
| b) | Explain the D.P.C. treatment across a RCC basement, with a neat cross-sectional sketch. |  |
| c) | Write any 4 considerations to be followed while painting a new masonry or plastered surface. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between the waterproofing technique for Bathrooms & Terraces. |  |
| b) | Write an elaborate note on the requirement of good plastering. |  |
| c) | Explain with a neat cross-sectional sketch, the water proofing treatment of an overhead water tank. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat cross sectional sketch, the construction of a concrete jack arch floor. |  |
| b) | Elaborate on the RCC beam & slab floor construction used in upper floor of a structure. |  |
| c) | Explain with sketches a double joist timber floor. |  |

***********