## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Automobile Engg.

Subject: Machine Tools & Production Process-II(4060) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Describe reaming operation. |  |
| b) | Name the different equipments used for mounting cutters in a horizontal boring machine. |  |
| c) | Give classification of Shapers. |  |
| d) | What is the function of Ram drive mechanism in slotting machine? |  |
| e) | Differentiate between Belt drive mechanism and hydraulic drive mechanism used in planning machine. |  |
| f) | Draw neat sketch of up milling and down milling processes. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the milling operation used for producing hexagonal surfaces. |  |
| b) | Draw a neat sketch of a column and knee type milling machine. |  |
| c) | Define cutting speed of a milling machine and calculate machining time to complete a cut on a milling machine given the following data: Feed per tooth = 0.1mm; RPM of cutter = 380rpm; Length of table travel to complete the cut = 310mm; Number of teeth on cutter = 40 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain hydraulic shaper mechanism with neat sketch. |  |
| b) | Explain machining of horizontal surface using Shaper. |  |
| c) | Explain any two work holding devices used in Shaper with neat sketch. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the process of gear cutting on a milling machine. |  |
| b) | Draw a neat labelled sketch of a Radial drilling machine. |  |
| c) | Explain any two work holding devices used in a drilling machine. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of a vertical boring machine with a neat sketch. |  |
| b) | How the size of a Planer is specified? Describe. |  |
| c) | List and describe the main parts of a Slotting machine. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Safety precautions while working on a drilling machine |  |
| b) | Jig boring machine |  |
| c) | Work holding devices used on a planning machine |  |
| d) | Slotter operation |  |
| e) | Milling cutters |  |

***********