## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mechanical Engg.

Subject: Engineering Metrology & Q.C.(ME406)/ Metrology & Q.C.(4047)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Distinguish between systematic and random error. |  |
| b) | Differentiate between tolerance and allowance. |  |
| c) | Distinguish between ‘Line standard’ and ‘End standard’. |  |
| d) | How do you measure flatness of a surface? |  |
| e) | Name the various elements of spur gear, which are inspected for accuracy of the gear. |  |
| f) | Explain the concept of quality circle. |  |
| g) | What is double sampling plan? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Taylors principle of gauge design |  |
| b) | With the help of a neat sketch, explain the principle of operation of a sine bar. |  |
| c) | Explain with neat sketch the principle of micrometer screw gauge. Also define its least count. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle of working of a dial indicator with a neat sketch. |  |
| b) | How would you measure chordal thickness and addendum using gear tooth vernier? |  |
| c) | Explain the concept of ‘OC’ curve. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the procedure of finding out minor diameter of screw thread. |  |
| b) | How squareness testing by dial indicator is done? |  |
| c) | How does the styles probe instruments work? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat sketch cost and value of quality. |  |
| b) | Explain the difference between control chart for variables and control chart for attributes. |  |
| c) | Control chart for X is to be prepared for a certain dimension of component. The subgroup size is 4. After 20 subgroups it is found that X= 825.60mm and R = 5.60mm. Compute the central line and the control limits for X-chart. d2 for sub-group size 4 is equal to 2.059. |  |
| Q.No.6. | Write short note on any four: | 4 x 3 = 12 |
| a) | Process capability |  |
| b) | Characteristics of good comparator |  |
| c) | Angle Gauges |  |
| d) | Screw thread micrometer |  |
| e) | Flatness testing by optical flats |  |

***********