## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2017 Examinations

Programme: Mining Engg.

Subject: Mining Technology(MN302) [Revised]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the different types of drilling methods? |  |
| b) | Write short notes on drag bits. |  |
| c) | What are the characteristics of a permitted explosive? |  |
| d) | Explain any three terms related to ground vibrations due to blasting. |  |
| e) | Differentiate between Friction & Hydraulic prop. |  |
| f) | Give the purpose of walling scaffold. What factors are considered while installing in a sinking shaft? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the characteristics of an emulsion explosive. |  |
| b) | - What are the essential constituents of an explosive? - Write short notes on Ammonium Nitrate. |  |
| c) | With a neat sketch explain the construction & composition of delay detonator. What are its advantages? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with neat sketch the procedure of drilling a wedge cut & pyramid cut. |  |
| b) | - What is deck charging? Explain the procedure. - What are the types of secondary blasting? Give advantages of each. |  |
| c) | The handling capacity of shovel is 2500 T of coal per day. Blasting is conducted every day. What is the blast design that you suggest with hole diameter of 125mm? Also calculate the powder factor of explosive whose density is 1.2 g/c.c. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & installation of a resin capsule roof bolt. |  |
| b) | - Differentiate between yielding & non-yielding supports. - What are the different means of testing roof? |  |
| c) | Give a neat sketch of any surface drill. Mention the purpose of each part. |  |
| Q.No.5. | Answer the following Questions: |  |
| a) | What are the different special methods of shaft sinking? Explain cementation method with a sketch. | (8) |
| b) | Mention the different methods of permanent lining. Give a brief note on any one. | (4) |
|  | ### OR |  |
| b) | Explain the long hole drilling method of raising. | (4) |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What parameters are considered in selection of drilling methods? |  |
| b) | How do you dispose old & damaged explosives? |  |
| c) | Explain the operation of Sylvester prop withdrawer. |  |
| d) | Draw a neat sketch of kibble. Explain how the mineral is unloaded. |  |