## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg.(CT)/C

Subject: Soil Mechanics(4012)/Soil Engineering(CT701) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | List out the points stating the importance of “Soil Mechanics”. |  |
| b) | Explain with neat sketch, soil mass as three phase system. |  |
| c) | Define the following terms with respect to soil mass: i) Porosity ii) Degree of saturation iii) Specific gravity |  |
| d) | Draw plasticity chart to explain soil classification. |  |
| e) | List the factors affecting permeability. |  |
| f) | Define Rankine’s theory and Coulomb’s theory of earth pressure. |  |
| g) | Write the concept of particle size distribution curve. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A soil sample has a porosity of 32%. The specific gravity of soil sample is 2.70. Calculate i) Void ratio ii) Dry density iii) Unit weight if soil is 50% saturated iv) Unit weight of soil if completely saturated. |  |
| b) | Explain the laboratory procedure for determining liquid limit by using casagrandes apparatus. |  |
| c) | With aid of neat sketch explain Active earth pressure and Passive earth pressure. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the determination of permeability coefficient by falling head permeameter. |  |
| b) | Explain how pore pressure on soil particles give rise to the quick flow condition. |  |
| c) | Explain Direct shear test to determine shearing strength. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain plate load test for determination of bearing capacity of soil. |  |
| b) | Explain the procedure for determining field density of soil using core cutter method. |  |
| c) | Explain in detail how bitumen stabilisation and cement stabilisation of soil is done. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain Electrical Resistivity method of soil exploration. |  |
| b) | Explain the method of obtaining undisturbed soil sample from the field. |  |
| c) | Explain any two methods of slope stabilisation against earth pressure of cuts and land fills. (only for students of revised scheme) |  |
| c) | What is uncofirmity? Draw neat sketch and explain Angular unconformity and discontinuity. (only for students of old scheme) |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain Sand replacement method. |  |
| b) | State the properties and uses of metamorphic rock. |  |
| c) | Derive relation between e, w, G and Sr. |  |
| d) | Explain Standard compaction test. |  |
| e) | State the typical values of bearing capacity of different soils & rocks in Goa. (only for students of revised scheme) |  |
| e) | Draw cross-section of Interior of earth and explain in brief. students of old scheme) | (only for |

***********