## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg./C/CT

Subject: Design of Structures-I(Steel)[4024)/Design of Steel Structures(FE701)

[Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of IS-800-1984 and steel tables is permissible.

| Q.No.1. | Answer any three of the following Questions: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Sketch typical stress-strain curve of steel, indicating the three important regions. |  |
| b) | List the hot-rolled steel sections used in practice and explain their application. |  |
| c) | What are the load combinations that are usually considered for truss analysis? Why is it necessary to provide connections that will allow movement in the supports of trusses? |  |
| d) | What is meant by effective length of a beam member? |  |
| e) | What is the major difference in behaviour between a joint with black bolts and a joint with HSFG bolts? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 150mm x 16mm plate is welded to other plate by two side welds 120 mm each and end fillet of 100mm length. Find the safe axial load to which this joint may be subjected if the size of the weld is 7mm. |  |
| b) | Design a double bolted lap joint for plates 16mm thick to carry its full load. Take permissible axial tension in plate 0.6 fy, where fy=250N/mm2. |  |
| c) | A tie bar consisting of a single angle 60mm x 60mm x 10mm is to be welded to a gusset plate. The tie bar carries a load of 150KN along its centroidal axis. Design the joint if both the side fillets and end fillets are to be provided. The centroidal axis of the angle lies at 18.5mm from the heel of the angle. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A member consisting of two ISA 90x90x10 back to back welded on one face of gusset plate are tack welded at 1000mm spacing and the assembly is subjected to tension. Determine the tension capacity of the member and weld required. |  |
| b) | Design an unequal angle section to act as a tie member 1.56m long in a roof truss if it is to carry an axial load of 120KN. Take shear stress = 108 MPa for weld. |  |
| c) | A member of a microwave tower is subjected to 350KN tension. Design the member using 24mm bolt connection. Determine the number of common bolts required to connect the member with a gusset. Try a single angle member. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Axial compressive force in a member of a truss is 50KN and the distance between its joints is 1.40m. Design the member using rolled section if fy=250N/mm2. -2- | …..2/- |
| b) | A 6m length column hinged at top and fixed at base is carrying an axial load of 400KN. Find ISHB section which can carry the load. |  |
| c) | A column of ISMB 400 is subjected to an axial force of 750KN. Design base plate. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a simply supported beam, having an effective span of 10m and carrying a uniformly distributed load of 40KN/m. The beam is laterally supported. Take fy=250KN/mm2 and E= 2 x 105 N/mm2 |  |
| b) | Compute the moment carrying capacity and shear strength of a laterally restrained ISMB 300 member of length 3m with fy = 250MPa. |  |
| c) | A cantilever beam of span 5m is subjected to a concentrated load of 22.5KN at its free end. Assume the beam to be laterally supported. Take fy= 250 MPa. There is no limit on deflection. |  |
| Q.No.6. | Answer the following Questions: |  |
| a) | Design a purlin for a roof truss having the following data: Span of truss = 12m Spacing of truss= 4m centre to centre | (10) |
|  | Inclination of the roof = 30 Spacing of purlin = 1.60m c/c Wind suction = 1.40 KN/m2 Weight of A.C. sheet roof coverage = 1.68 KN/m2 Live Load = 0.75 KN/m2 of plan area less 0.02 KN/m2 for every degree increase in slope over 10 (minimum 0.40 KN/m2) ii) Provide channel purlin iii) Provide angle purlin and design by empirical method |  |
| b) | Explain how internal and external wind pressure on trusses are calculated. | (2) |

***********