## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-I(4037) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Write down any three advantages of wind energy. |  |
| b) | What do you understand by higher calorific value and lower calorific value? |  |
| c) | State and explain the Boyle’s law. |  |
| d) | Define the following terms: i) Sensible heat of water ii) Dryness fraction of steam |  |
| e) | Explain briefly what you mean by entropy. |  |
| f) | Write down any three points of difference between water tube and fire tube boilers. |  |
| g) | Write down two points of difference between a fan and a blower. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short notes on: i) Fossil energy ii) Biogas. |  |
| b) | Explain briefly the working of the Orsat apparatus for flue gas analysis with help of a neat sketch. |  |
| c) | Explain i) Isochoric process ii) Isentropic process on PV and TS diagrams. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A quantity of gas has a volume of 0.14m3, pressure 1.5 bar and a temperature 100°C. If the gas is compressed at a constant pressure until its volume becomes 0.112m3, determine i) The temperature at end of compression ii) Work done in compressing the gas iii) Decrease in internal energy. Take Cv = 0.712 KJ/KgK; R = 285J/Kgk. |  |
| b) | Explain with a neat sketch the working of a separating calorimeter. |  |
| c) | Calculate the internal energy of 1Kg of steam at a pressure of 10 bar when the steam is i) 0.9 dry and ii) dry saturated. The volume of water may be neglected. From steam tables, corresponding to a pressure of 10 bar, hf = 762.6 KJ/Kg, hfg = 2013.6 KJ/Kg and g = 0.1943 m3/Kg. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain modified Rankine cycle with help of PV and TS diagrams. |  |
| b) | i) What is the function of a safety valve in boilers? ii) Explain briefly the working of a blow off cock. | (2) (4) |
| c) | Explain briefly with a neat sketch the working of a Cochran boiler. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain briefly with a neat sketch the working of a Superheater. |  |
| b) | With help of a neat sketch explain the working of a single stage reciprocating compressor. -2- | ……2/- |
| c) | Explain briefly with help of a neat sketch the working and construction of an axial flow compressor. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Write a short note on ‘Applications of compressed air’. |  |
| b) | Explain the following terms: i) Specific heat at constant volume ii) Specific heat at constant pressure |  |
| c) | Write a short note on ‘Mollier chart’. |  |
| d) | Write a short note on ‘Boiler draught’. |  |

***********