## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(AT402) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by TDC & BDC? Explain clearance volume. |  |
| b) | Give four points of difference between SI and CI engine. |  |
| c) | Sketch a camshaft and list out the functions of it. |  |
| d) | Explain any two possible causes of engine overheating |  |
| e) | What is crankcase ventilation? How is it achieved in an automobile engine? |  |
| f) | Discuss the various steps which should be taken to prevent detonation in IC engine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the functions of engine inlet and exhaust manifolds? Describe their materials and constructions. |  |
| b) | Discuss different types of cylinder liners. What are their comparative advantages? Discuss various liner troubles. |  |
| c) | Write explanatory note on Guolgeon pin. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the main components of water cooling system for an automotive engine? Explain in brief with sketch. |  |
| b) | Discuss the common faults occurring in the engine cooling system and also suggest suitable remedies. |  |
| c) | What are the merits and demerits of an air cooled engine? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch showing the components of a dry-sump method of engine lubrication. Explain its working. |  |
| b) | What are the objectives of using an oil cooler? Explain briefly the construction and working of a water type oil cooler. |  |
| c) | List out the oil properties taken into consideration while selecting engine lubricant. Explain what is “Viscocity index”. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is gasoline? Discuss its properties. Also list out other fuels which are fit for automotive use. |  |
| b) | With the help of a sketch explain the principle of working of diesel fuel feed system. |  |
| c) | Describe briefly the following phenomena: - Pre-ignition ii) Diesel knock |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What do you mean by vibration damper? List out its functions. |  |
| b) | What are the functions of a radiator in cooling system? |  |
| c) | With the help of a neat sketch, explain construction of an oil strainer. |  |
| d) | Write a brief note on Octane number of fuel. |  |

***********