## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/E/E&EE

Subject: Electronic Instrumentation(4141)/(EN404) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer the following Questions: |  |
| --- | --- | --- |
| a) | Differentiate between active and passive transducer. | (2) |
| b) | Give one example each for analog and digital transducer. | (1) |
| c) | Name any two materials used for thermoelectric transducers. | (1) |
| d) | Explain measurement of linear displacement using potentiometer. | (3) |
| e) | What is signal conditioning? Why is it required? | (2) |
| f) | Draw the block diagram of a process control system. | (2) |
|  | Only for students of Revised scheme: |  |
| g) | Write one advantage and one disadvantage of LED displays. | (1) |
| h) | List the application areas of Data Acquisition System. | (3) |
|  | Only for students of Old scheme: |  |
| g) | List two characteristics of Data Logger. | (1) |
| h) | Draw block diagram of Data Logger. | (3) |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the block diagram of an instrumentation system and explain the function of each block. |  |
| b) | i) With a neat diagram, explain the construction of a Linear Potentiometer. ii) With a neat diagram, explain the construction and operation of a bonded wire strain gauge. |  |
| c) | Explain the working principle of an Inductive transducer based on variation in self inductance. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | With neat diagrams explain the construction and operation of Thermistor. |  |
| b) | Explain the working of a capacitive transducer based on the variation of distance between the plates. |  |
| c) | Explain the working of Thermocouple as a transducer. |  |
| d) | Explain the working of a photovoltaic cell as a transducer. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain measurement of linear and angular displacement using any one method. |  |
| b) | With a neat diagram explain measurement of liquid level using float operated resistive transducer. |  |
| c) | With a neat diagram explain the working of a turbine flow meter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain angular speed measurement using photoelectric pickup. -2- | ……2/- |
|  | Only for students of Revised scheme: |  |
| b) | Explain pressure measurement using strain gauge with diaphragm. |  |
| c) | With a neat block diagram explain the function of a SCADA system. |  |
| d) | What are the applications of SCADA? Write briefly on any three applications areas of process control system. |  |
|  | Only for students of Old scheme: |  |
| b) | Explain pressure measurement using Vacuum Gauge. |  |
| c) | Draw and explain the block diagram of a Data Acquisition System. |  |
| d) | Explain in detail any one Composite Controller Mode. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram explain the function of an instrumentation amplifier using OPAMPs. |  |
|  | Only for students of Revised scheme: |  |
| b) | Briefly describe the working of an X-Y plotter. |  |
| c) | With a neat circuit diagram explain a bridge circuit used along with RTD. |  |
|  | Only for students of Old scheme: |  |
| b) | Explain the elements of Final Control Operation. |  |
| c) | Explain the working of AC Servo Motor. |  |
| d) | Explain the 2-position Discontinued Controller Mode. |  |

***********