## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Piping Fabrication & Installation(5342)/(FE611) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Refer data given at the end.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With the help of a neat sketch, explain how insulation for a hot pipeline is done. |  |
| b) | Explain pipe schedule number, nominal diameter and class/grade of a pipe. |  |
| c) | Briefly discuss the following standards used in piping system design: (i) ASME (ii) API (iii) ASTM |  |
| d) | Draw single line symbols for following fittings: (i) 90° Elbow (ii) Reducer (iii) Blind |  |
| e) | Why is flexibility needed in pipelines? |  |
| f) | A gas pipeline is installed in a filling station. What are the special considerations for laying this pipeline? |  |
| g) | Explain piping edge preparation for welding. |  |
| Q.No.2. | Design a pipe of material ASTM A106 grade B to fabricate a pipeline to conduct a fluid at 200°C, delivery pressure of 10 Kg/cm2 and discharge if 20 GPM. The total length of the pipeline is 300m long and has the following fittings incorporated: (i) Gate valves – 02 (ii) Globe valves – 01 (iii) Long radius elbows -10 (iv) Tee – 02. | (12) |
| Q.No.3. | Answer the following Questions: |  |
| a) | Draw the isometric view and develop view of the portion of pipeline, whose views are shown in fig.1 (single line) | (8) |
| b) | Which factors are considered for deciding material of construction of a pipeline? | (4) |
|  | ### OR |  |
| b) | Explain any one method of cold bending of a pipe. | (4) |
|  | -2- | ….2/- |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the use of following drawing types: i) Process flow diagram ii) Piping & Instrumentation diagram |  |
| b) | How do we calculate the thickness of a pipe for a particular use? |  |
| c) | For a  100mm, outside diameter pipe, calculate the minimum length of pipe required to make a 4-pieces, 90° bend. (only for students of revised scheme) |  |
| c) | Draw sketches of any 4 type of pipe supports. (only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly discuss the importance and use of corrosion tables. |  |
| b) | Explain the purpose and steps involved in hydro-testing of pipelines. |  |
| c) | Write a note on ERW pipes manufacturing. (only for students of revised scheme) |  |
| c) | Write the list of drawings used in any process industry for piping systems. (only for students of old scheme) |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Special design consideration for toxic pipelines |  |
| b) | Welder qualification |  |
| c) | Clearances and tolerances in piping work |  |
| d) | Effect of velocity of flow with change in size of pipe (only for students of revised scheme) |  |
| d) | Stress v/s temperature rise in pipelines (only for students of old scheme) |  |

Table 1: Equivalent length of fittings

| Sr.No. | Item | ‘Le’ – Equivalent length (ft) |
| --- | --- | --- |
| 1 | Gate Valve | 1.00 |
| 2 | Globe Valve | 40.00 |
| 3 | Long radius 90 Elbow | 3.00 |
| 4 | Tee | 3.00 |

Table 2: Allowable stresses of ASTM A106 Gr.B

| Temp. F | -20 to 650 | 700 | 750 | 775 |
| --- | --- | --- | --- | --- |
| Allowable stresses ‘s’ psi | 15,000 | 14,300 | 12,900 | 11,850 |

…..3/-

-3-

Table 3: Pressure drop per 100 ft of Sch 40 pipe

| GPM | Pipe size NB |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 1¼  | 1½  | 2 |
| ZO | 14.42 | 3.66 | 1.69 | 0.486 |

Table 4: Pipe properties (Sch 40)

| Pipe size (NB) | O.D. (inch) | I.D. (inch) | Wall thickness (inch) |
| --- | --- | --- | --- |
| 1 | 1.315 | 1.049 | 0.133 |
| 1¼  | 1.660 | 1.380 | 0.140 |
| 1½  | 1.900 | 1.610 | 0.415 |
| 2 | 2.375 | 2.067 | 0.154 |