## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Fluid Machinery(4040) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A jet of water of diameter 30mm moving with a velocity of 20 m/s, strikes a curved fixed symmetrical vane at the centre. Find the force exerted by the jet if it is deflected through an angle of 120º at the exit of the curved vane. |  |
| b) | Differentiate between impulse and reaction turbine. |  |
| c) | State three advantages of centrifugal pump over reciprocating pump. |  |
| d) | Give the classification of reciprocating pump. |  |
| e) | State the applications of vane pump. |  |
| f) | Give the function of shuttle valve in pneumatic control system. Also show its symbol. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following data pertains to a single jet Pelton turbine: shaft power=130KW; Head=300m; Speed= 650 r.p.m; coefficient of velocity(Cv)=0.98; speed ratio(Ku)= 0.45; Overall efficiency = 75%. Find the i) Discharge ii) Runner diameter iii) jet diameter. |  |
| b) | Draw a schematic diagram of a Kaplan turbine and explain its working. |  |
| c) | Explain the phenomenon of cavitation occurring in turbine. List the methods to avoid cavitation in water turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch and discuss the operating characteristics curves of turbines. |  |
| b) | Explain with neat sketches the following parts of a centrifugal pump: i) Vortex casing ii) Closed type of impeller. |  |
| c) | A three stage centrifugal pump has impellers 300mm diameter and 21.5mm width at outlet. The vanes are curved back at an angle of 30º to the tangent at outlet. The pump delivers 40 lit/s of water at 1000 r.p.m. with manometric efficiency of 85%. Calculate the total head generated by the pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly explain the different factors to be considered in the selection of centrifugal pump. |  |
| b) | Show that there is a considerable saving of work by fitting air vessel in a single acting reciprocating pump. |  |
| c) | With the help of a neat sketch explain the construction and working of double acting reciprocating pump. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of screw pump. |  |
| b) | Classify the different types of direction control valves in pneumatic system. Explain the working of any one type of direction control valve with the help of a neat sketch. -2- | …….2/- |
| c) | With the help of a neat sketch explain the meter out type of hydraulic control circuit. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is an unloading valve? Describe its working in hydraulic system with a simple sketch. |  |
| b) | Write a short note on maintenance of pneumatic control system. |  |
| c) | Draw the ideal indicator diagram for a single acting reciprocating pump. Establish the correlation between indicator diagram and work done. |  |

***********