## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(4339)/(SH601) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Froude’s Number |  |
| b) | Eddy making resistance |  |
| c) | Pitch of a propeller |  |
| d) | Thrust deduction factor |  |
| e) | Controllable pitch propeller |  |
| f) | Balanced rudder |  |
| Q.No.2. | Answer the following: |  |
| a) | Explain the purpose and procedure of model testing and state the similarities maintained in model testing. | (6) |
| b) | A ship with wetted surface area of 5150m2 travels at 15 Knots. Calculate the frictional resistance if Froude’s friction constant f = 0.422 and power of speed n = 1.825. Also determine the Froude number if the length of vessel is 146m. | (6) |
| Q.No.3. | Answer any one of the following: | (12) |
| a) | A model 5.2m in length has total resistance of 67 Newton when towed at 3.5 Knots. The wetted surface of the model is 4.25m2. Determine the effective power for the ship of similar form and having length 131m at the corresponding speed. fmodel = 1.728 sw fship = 1.418 sw |  |
| b) | Explain the terms: i) Quasi propulsion co-efficient (QPC) ii) Open water efficiency iii) Relative rotating efficiency iv) Transmission efficiency. |  |
| Q.No.4. | Answer any one of the following: | (12) |
| a) | A propellor has the diameter of 4.6m. At 110 rev/min the speed of the ship is 16 Knots. The propellor has efficiency of 68%, Froude wake friction 0.22 and Thrust deduction fraction 0.15. Determine PE, PT, QPC and actual thrust for a delivered power of 1820 KW. |  |
| b) | What is a screw propellor? Draw a typical propeller drawing and explain the geometry. |  |
| Q.No.5. | Answer the following: |  |
| a) | Explain any one of the below mentioned special propellor types with salient features, advantages and disadvantages. - Vertical axis propellor - Super cavitating propellor | (6) |
| b) | Explain with a diagram the oil cooled stern tube. | (6) |
| Q.No.6. | Answer the following: |  |
| a) | Describe briefly the maneuvering trials carried out on ships. | (6) |
| b) | When does a ship need to be provided a reduction and reversing gear box? Explain the functioning of reversing and reduction gear box. | (6) |

***********