## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(4026)/(CE504) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the concept of composite section. |  |
| b) | Explain middle quarter rules. |  |
| c) | A simply supported beam of span 3m is subjected to a central load of 10KN. Find the maximum deflection. I = 12 x 106 mm4 and E = 200 x 103 N/mm2 |  |
| d) | What do you understand by column and strut? Explain Short column. |  |
| e) | Define the terms ‘Carry over factor’ and ‘Stiffness factor’. |  |
| f) | What is meant by an encastre beam? Is there any advantage in using it? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A flitched beam consists of two wooden joists of 100mm wide, 200mm deep with a steel plate 140mm deep and 10mm thick placed symmetrically between them. If maximum stress in wooden joist be 7 N/mm2, find the corresponding maximum stress reached in steel. Find also moment of resistance of section. Take Es=20Ew. |  |
| b) | A column 75mm x 75mm has a circular cavity of 38mm in diameter at the centre. Find the maximum value of load that can be applied at an eccentricity of 12mm with respect to XX axis in order that maximum compressive stress may reach 80N/mm2. |  |
| c) | A column section 400mm external diameter and 200mm internal diameter supports an axial load of 4000KN and an eccentric load of 800KN at an eccentricity of 400mm. Find the intensities of maximum and minimum stress along the diameter. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular block of material is subjected to a tensile stress of 100 N/mm2 on one plane and a tensile stress of 50 N/mm2 on a plane at right angles, together with shear stress of 60 N/mm2 on the same plane. Find i) Magnitude of principal stress ii) Magnitude of the greatest shear stress. |  |
| b) | At a point in an elastic material, a direct tensile stress of 60 N/mm2 and a direct compressive stress of 40 N/mm2 are applied on planes at right angles to each other. If the maximum principal stress is limited to 75 N/mm2(tensile), find the shear stress that may be allowed on the planes. Also, determine the minimum principal stress and maximum shear stress. |  |
| c) | A simply supported beam of span 6m carries a point load of 60KN acting at 2m from left support. The size of the beam is (300 x 350)mm. Find the maximum slope and deflection in the beam. E = 0.15 x 105 MPa |  |
|  | -2- | …..2/- |
| Q.No.4. | Answer the following Questions: |  |
| a) | A simply supported beam AB of span 4m carries a UDL of 8 KN/m for entire span. Find the maximum deflection. Take EI = 1.25 x 1012 N-mm2. | (2) |
| b) | A horizontal beam of uniform section and 6m long is simply supported at its ends. Three vertical concentrated loads 48KN, 40KN, 55KN act at 1m, 3m, 4m respectively from the left support. Find the deflection under 40KN load using Macaulay’s method. E=200 GPa and EI = 85 x 106mm4. ### OR | (10) |
| c) | A fixed beam of span 8m carries a two point loads of 45KN and 75KN at 2m and 4m from left support. Calculate the support moments and draw SFD & BMD. | (10) |
| Q.No.5. | Answer the following Questions: |  |
| a) | A built in beam of span 4m carries a UDL of 15 KN/m. Find the fixed end moments. Also find the maximum positive moments. | (5) |
| b) | A continuous beam supported at ABC, such that AB=6m, BC=5m. The span AB carries UDL of 30 KN/m and BC carries a point load of 50KN at 2m from support B. Take IAB = 2I and IBC = 3I. Using moment distribution method, find the moments and draw SFD & BMD. OR | (7) |
| c) | A symmetrical portal frame ABCD consists of columns AB and CD = 5m and beam BC = 4m. The beam BC carries a point load at the centre. Take IAB=ICD=2I and IBC=3I. Supports A and D are fixed. Find the moments and draw SFD & BMD. | (7) |
| Q.No.6. | Answer the following Questions: |  |
| a) | A steel rod 5m long and of 40mm diameter is used as a column with one end fixed and other end free. Find the crippling load by Euler’s formula. E = 200 x 103 N/mm2 | (5) |
| b) | An I section joist 400mm x 200mm x 20mm and 6m long is used as a strut with both ends fixed. What is Euler’s crippling load for the column? Take Young’s modulus of the joist as 200 x 103 N/mm2. OR | (7) |
| c) | A hollow column of 200mm external diameter and 160mm internal diameter is used as a column of 5m length. Calculate Rankine’s crippling load when the column is fixed at both the ends. Take the allowable stress as 350Mpa and Rankin’s constant as  = 1/1600. | (7) |

***********