## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mining Engg.

Subject: Mine Ventilation(MN503) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How does carbon monoxide affect the humans on inhalation? |  |
| b) | Differentiate between central and boundary ventilation. |  |
| c) | Explain the terms: Antitropal ventilation and Descensional ventilation. |  |
| d) | On what factors does resistance of gallery depend upon? |  |
| e) | What are the different ways of increasing air velocity in axial flow fans? |  |
| f) | Write short note on flexible ducts. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principal and construction of flame safety lamp. |  |
| b) | i) Write short note on Methane layering. |  |
|  | ii) What do you understand by guage pressure & absolute pressure? |  |
| c) | What are the different sources of heat in an underground coal mine? How does humidity affect the performance of worker? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction & installation of a ventilation door. Under what conditions a double door is installed? |  |
| b) | Explain with a neat sketch overlapping method of ventilation of narrow headings using auxiliary fans. |  |
| c) | What is Motive column? A downcast shaft is 200m deep having average temperature of 18ºC. The upcast shaft is 150m deep having average temperature of 26ºC. Calculate the motive column. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the purpose of splitting? Give its advantages and disadvantages. |  |
| b) | Explain the concept of equivalent orifice. Total quantity of air passing through a district of a mine is 30 m3/sec with a water guage of 20mm. It is now proposed to reduce the quantity of air flowing to 25 m3/sec. Calculate the size of the regulator that should be fitted in the district. |  |
| c) | Three splits in parallel, of similar cross-section and same type of roadway surface are respectively 300m, 600m and 900 m long. Calculate the quantity of air which should flow in each if the total quantity is 600 m3/min. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the principal & construction of a centrifugal fan. |  |
| b) | What factors are considered in design of an evasee? The evasee chimney of a fan has an area of 4m2 at the base and 16m2 at the outlet. Calculate the saving of w.g. due to chimney when the output of the fan is 6000 m3/min. Assume air density to be 1.2 Kg/m3. -2- | …..2/- |
| c) | A 4m diameter backward bladed centrifugal fan delivers 8000 m3/min when running at 300 rpm. If the blade angle is 35º and width of fan at periphery is 1.2m, calculate the theoretical w.g. developed by the fan. If the actual w.g. obtained is 280, calculate the manometric efficiency. Assume density of air to be 1.2 Kg/m3. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the objectives of mine ventilation? |  |
| b) | How is quantity ventilation survey done? |  |
| c) | Draw a neat sketch of characteristic curves of any fan & explain the salient features. |  |
| d) | Explain the principle of Pitot-static tube. |  |

***********