## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT/M

Subject: Hydraulics(4025)/(CE405) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the terms capillarity, viscosity and surface tension. |  |
| b) | What is a Monometer? What is its working principle? |  |
| c) | Compare Laminar and Turbulent Flow. Give one example of each. |  |
| d) | How does a Cippoletti weir differ from a Rectangular sharp crested weir? |  |
| e) | What is Hydraulic Gradient Line and Total Energy Line? |  |
| f) | Define Velocity of Approach and how is its calculated in case of flow in open channels. |  |
| g) | Distinguish between Centrifugal pump and Reciprocating pump. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the working of simple U-Tube Manometer with a neat sketch. |  |
| b) | A rectangular plate with sides of 3m each is immersed in a liquid of specific gravity 0.85 such that the base of plate is in level with free surface of the liquid. Find the total pressure and centre of pressure. |  |
| c) | An Inverted Differential Manometer containing an oil of specific gravity 0.9 is connected to find the difference of pressures at the two points of pipe containing water. If the monometer reading is 40 cms, find the difference in pressures at the two points. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A 150mm x 75mm Venturimeter is connected in a pipe discharging oil of specific gravity 0.85, which is inclined at an angle of 45º with the horizontal. Find the discharge through the Venturimeter, if the memory gauge shows a deflection of 180mm and coefficient for venturimeter is 0.96. |  |
| b) | Define Orific, Mouthpiece and Vena Contracta. Mention Hydraulic coefficients and write the formula for each. |  |
| c) | Determine the most economical section of a rectangular channel carrying water at the rate of 0.5 m3/s. The bed slope of the channel is 1 in 2000. Take Chezy’s constant C=50. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rectangular notch 3m wide has a constant head of 50cm. Determine the discharge over the notch if coefficient of discharge for the notch is 0.64. |  |
| b) | Find the discharge through Trapezoidal Notch which is 1m wide at the top and 0.4m at the bottom and 30 cm in height. The head of water on the notch is 0.2m. Take cd = 0.62 (Rectangular portion) and cd = 0.6 (For Triangular portion). -2- | …….2/- |
| c) | A vertical tank square in plan having side width 2m and 3.5m height contains three liquids of specific gravity 1.2 at bottom 1m height, water (s.g. 1.0) in the middle 1m height and oil of specific gravity 0.8 at top 1m height. Determine the total pressure on one vertical side of tank using pressure diagrams. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Three pipes of diameter 300mm, 200mm and 400mm having length 450m, 255m and 315m respectively are connected in series. The difference in water surface in two tanks is 18m. Determine rate of flow if coefficient of friction are 0.0075, 0.0078 and 0.0072 respectively considering major losses only. |  |
| b) | A rectangular channel is to be dug in the rocky portion of soil. Find it most economical cross section if it is to convey 12 m3/sec of water with an average velocity of 3 m/s. Take Chezy’s constant as 50. |  |
| c) | Define the term Water Hammer. What are its causes & effects and suggest some measures to reduce Water Hammer. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Hydraulic Jump |  |
| b) | Re-entrant Mouth Piece |  |
| c) | Selection of Pump |  |
| d) | Hydraulic Ram |  |
| e) | Types of Mouth Pieces |  |

***********