## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Automobile Electrical & Electronics(AT601)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Discuss the discharging process of a lead acid battery. |  |
| b) | Write a comprehensive note on the maintenance of Batteries. |  |
| c) | Explain wiring harness and gives 3 advantages of the same. |  |
| d) | Explain the need for using the over-running clutch drive in a starting system. |  |
| e) | Draw a block diagram of the starting system and list out the engine starting requirements. |  |
| f) | What is the function of condenser in the ignition system? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is battery rating? Explain different types of battery ratings. |  |
| b) | Describe the following with sketches: i) The plate group ii) Plate separator iii) Battery container |  |
| c) | Explain what is meant by sulphation? Give its causes and remedies. Explain the various causes of failure of lead acid battery. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by cable color codes? Give the types of automobiles cables and explain any one in detail. |  |
| b) | With the help of a neat sketch explain circuit diagram of ignition warning light. |  |
| c) | Describe with sketch Headlight dazzle and what are the measures taken to prevent it. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is an alternator? Compare it with d.c. generator. Which will you prefer? Give supporting reasons for your answer. |  |
| b) | List out 4 possible failures of Alternator and provide its necessary corrections. |  |
| c) | Explain clearly the working principle of starter motor. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain the operation of a Bendix drive. |  |
| b) | With the help of a neat diagram explain Battery Ignition system. |  |
| c) | With the help of a neat block diagram, explain the working of a distributerless Ignition system. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain how the alternator ouput is rectified. |  |
| b) | With neat diagram explain the operation of starting solenoid. |  |
| c) | Write a note on heat ranges of spark plug. |  |
| d) | Explain the functions of an ignition coil and why is the primary winding wound outside compared to the secondary windings. |  |

***********